A Virtual Distillation Column used in

Engineering Education and Research

# Seppo Lahti and Ingmar Tollet

Espoo-Vantaa Institute of Technology

Vanha maantie 6

FIN-02600 Espoo, Finland

7.   ADVANCED CONTROL 1, 8

The robustness approach is based on a methology which systematically takes into account uncertainties of any kind. There are two types of uncertainty and each one leads to its own methodology. The uncertainty is Unstructured Uncertainty ( the leads to Hinf  Control Methology ) and the uncertainty is Structured Uncertainty ( this lead to Parametric Control Methology ). We will be treating in our matters only Unstructured Uncertainty, hence only this case will be considered. As reference control approaches we are using standard PID algorithms.

The following well- known approaches will be considered:

1)   Least-Squares Control ( LQG, LQG/LTR,... ):

H2 Optimal Control:	min // Tzw //2

K ( s ) stabilising

2)   Canonical Robust Control:

Standard Hinf  Control:	Find // Tzw //inf < 1

K ( s ) stabilising

( The Standard Hinf Control Problem is sometimes also called the Hinf  Small Gain Problem )

Hinf Optimal Control:	min // Tzw //inf

K ( s ) stabilising

The K ( s ) is the controller and the T transfer function between z and w is received by transforming the original system appropriately. For details, we refer to Skogestad & al.

8.   SIMULATION STUDIES

The closed - loop control set-up in terms of physical equipment and the corresponding mathematical model to be investigated is exhibited in the next two figures. Hence in our simulations we will be using this latter set - up, i. e. the transfer functions Gp ( s ) ( if the process is linear ) and Gc ( s ) will be specified for each particular case. The water level, i. e. the height h, is controlled.

In our control case the variable to be changed, i. e. to be manipulated is the input flow fi .. .

Figure 4.   The liquid tank is exhibited as a simple example

Figure 5.   The block diagram of the liquid tank

The simulation of the linear tank is done based upon the Figure 6 below, i. e. a graphic

set - up with all blocks, which are linear, are given as Laplace transform functions.

Figure 6.   The SIMULINK model of the linear tank

For the linear case, as given above, the transfer functions are given as numerator over denumerator. However, when the blocks cannot be expressed in this form, the detailed structure in terms of suitable blocks have to be used.

PIDSIMU/MATLAB: VDC98 Case 1, td = 1.

Figure 7.  ( NO1FIG1.DOC )	Case 1 ( Linear )

PIDSIMU/SIMULINK: VDC98 Case 1, td = 1.

Figure 8.  ( NO1FIG1N.DOC )	Case 1 ( Non-linear )

´9.   CONCLUSIONS AND FURTHER DEVELOPMENTS

The first phase of the Virtual Distillation Column has been implemented in the Internet

( htpp://pww.evitech.fi/vdc/ ). The first phase means that the basic Web pages, the tutorial material consisting of the linear tank, the non-linear tank and the distillation column have been separately implemented inclusive general matters with respect to the Web. The control schemes have been implemented, but only the PID approach is exhibited in the results given here. Also, only the non-linear tank is alive in the Web, but it has been implemented as a separate program independent of the MATLAB/SIMULINK system.

The next steps will be the implementation of the interface between the Web and the MATLAB/SIMULINK environment, the Neural Network approach for modelling and the distillation column itself both as a non-linear and a linearised model of the process.

10.   REFERENCES

1.   Skogestad S. and Postlethwaite I., Multivariable Feedback Control, Wiley, 1996

2.   Seppo Lahti, Seppo Haltsonen and Ingmar Tollet, A Virtual Distillation Column, totally or only

partially, used in Engineering Education Environments, International Symposium “ Engineering

Education ‘98 “, Moscow, 14 - 18 September 1998

3.   MATLAB Application Program Interface Reference. Version 5. The Mathworks, 1998

4.   Max Demenkov and Alexandre Gavrilov, Basic Principles of MATLAB/WEB

Interconnection   Bauman, Moscow, 1998

5.   Hämäläinen, M. and Rämö, E. (1997) Real Life Learning and Education in Virtual Networks.

Proceedings of EDMEDIA97 - A World Conference on Educational Multimedia/Hypermedia

and Telecommunications, Calgary, Canada, June 1997, 421 - 428

6.   Hämäläinen, M. (1996) A Model for Delivering Customised On-demand Education on the Internet.

In: A. Dumort and W. Paprotte (eds) The Road to the Information Society. New Technologies for

Education and Training. European Commission, DGXIII. Luxembourg, 104-125

7.  Gavrilov, A., NNPredictor - the MATLAB - based Toolkit, Version 1.4., NeuroCybernetics Lab.,

BMSTU, Moscow

8.  Chiang R. and Safonov M., Robust Control TOOLBOX, User’s Guide, The MATH WORKS, 1992