### Solving linear differential equations using the Laplace transforms

The method of Laplace transforms, that is described below is somehow the best suited for solving the type of problems given by Equation (1.1) (linear continuous time-invariant models) which are of interest in general for control engineers. In many ways the Laplace transform method is similar to the preceding method of using the partial-fraction expansion to reduce the nth order equation to the sum of n lower-order equations. A major difference is that, in the Laplace transform method, the response due to each term in the partial-fraction expansion is determined directly from the transform table. Thus there is no need to perform the integration indicated by either Equation (1.29) or Equation (1.36). The initial conditions are automatically incorporated into the solution, so the resulting response expression yields directly the total solution (i.e., complementary plus particular solution). Thus, the constants arising from the initial conditions are automatically evaluated and the final desired result is obtained directly.

The process of finding the solution of (1.1) consists of three main steps:

1., The given differential equation is transformed into a „simple“ algebraic equation of variable s (the Laplace transforms’ variable) by using the direct Laplace transform.

2., The algebraic equation is solved by pure algebraic manipulations.

3., The solution of algebraic equation is transformed back (using the inverse Laplace transform) to obtain the solution in time-domain of the given model.

We have to note here that in control engineering the information obtained in s-domain after applying the Laplace transform to Equation (1.1) suffices in many cases, so that it may be unnecessary to invert back to the time-domain.

#### The Laplace transform

Given the function x(t) the Laplace transform of x(t) is defined as:

Xs=Lx(t)=x(t)e−stdt,	(1.37)

where L is the symbol of taking the Laplace transform. To perform the integral, the function x(t) must be defined and piecewise continuous on every finite interval in the range t³0 and must satisfy x(t)≤Me for all t³0 and for some constants M and g.

The variable s is referred to as the Laplace transformation variable or the Laplace operator, so the Laplace transform can be either represented as:

xtLXs,	(1.38)

The s variable is a complex number, that is:

s=s+jw.	(1.39)

The defining Equation (1.37) is also known as one-sided Laplace transform, as the integration is evaluated from 0 to ¥. This simply means that all information contained in x(t) prior to t=0 is ignored or considered zero. This assumption does not place any serious limitation on the application of the Laplace transform to linear system problems, since in the usual time-domain studies the time reference is often chosen at the instant t=0. Furthermore, for physical systems when an input is applied at t=0, the response of the system does not start sooner than t=0; the physical system is causal, that is the response does not precede the excitation.

In control system applications some input functions of system are frequently used for investigation. Their Laplace transform pairs are collected in Table 1.2.

Table 1.2. Laplace transform pairs

| ftxt | FsXs |
| --- | --- |
| dt- unitary impulse | 1 |
| 1t- unitary step | 1s |
| tn | n!sn−1 |
| eat | 1s−a |
| tneat | n!(s−a)n−1 |
| sinwt | ws2+w2 |
| coswt | ss2+w2 |
| eatsinwt | w(s−a)2+w2 |
| eatcoswt | s−a(s−a)2+w2 |
| 12w3(sinwt−wtcoswt) | 1(s2+w2)2 |
| t2wsinwt | s(s2+w2)2 |
| 12w(sinwt+wtcoswt) | s2(s2+w2)2 |

#### The inverse Laplace transform

The operation of obtaining xt from the Laplace transform Xs is termed the inverse Laplace transformation. The inverse Laplace transformation of Xs is denoted by:

xt=L−1Xs=12pjXsestds,	(1.40)

where c is a real constant that is greater than the real parts of all the singularities of Xs. We have to remember that a singular point of a complex function is the point where the function ceases to be analytic. Equation (1.40) represents a line integral that is to be evaluated in the s-plane (complex plane). However, for most engineering purposes this integral is not employed; the inverse Laplace transform operation can be accomplished simply by referring to the Laplace transform table. Such tables are given in almost every system theory or control engineering book.

#### Important theorems (properties) of the Laplace transform

The applications of Laplace transform in many instances are simplified by the utilisation of the properties of the transform. In the following these properties are presented shortly as theorems, without proofing them.

1. Linearity of the Laplace transform

The Laplace transformation is a linear operation: for any functions x1t and x2t whose Laplace transforms exist and for any constants c1 and c2:

Lc1x1t+c2x2t=c1Lx1t+c2Lx2t=c1X1s+c2X2s.	(1.41)

2. Laplace transform of the derivative of xt

The Laplace transform of the first derivative of a function xt with respect to t is s times the Laplace transform of xt minus the limit of xt as t approaches 0+; that is:

Ldx(t)dt=sXs−x(0+).	(1.42)

The theorem can be generalised. That is, for higher order derivatives:

Ldnx(t)dtn=snXs−limt→0sn−1x(t)+sn−2dx(t)dt+K+dn−1x(t)dtn−1=snXs−sn−1x(0+)−sn−2x(0+)−K−x(n−1)(0+).	(1.43)

3. Laplace transform of the integral of xt

The Laplace transform of the first integral of a function x(t) with respect to t, is the Laplace transform of x(t) divided by s; that is:

Lx(t)dt=Xss .	(1.44)

In general, for nth-order integration, one get:

LKx(t)dtn=Xssn .	(1.45)

4. Laplace transform of a t-shifted function

The Laplace transform of a function xt shifted by a real value T0 is equal to the Laplace transform of xt multiplied by e−T0s, that is:

Lxt−T0=e−T0sXs	(1.46)

The property is also known as real translation.

5. Multiplication by t

The Laplace transform of a function xt multiplied by its variable t is:

Ltxt=-ddsXs .	(1.47)

In general, one can write:

Ltnxt=(−1)ndndsnXs .	(1.48)

(The last three transforms from Table 1.2. were obtained using this theorem.)

6. Division by t

The Laplace transform of a function xt divided by its variable t is:

Lxtt=Xsds .	(1.49)

The Formula (1.49) can be also easily extended.

7. Multiplication by eat

The Laplace transform of a function xt multiplied by eat is:

Leatxt=Xs−a.	(1.50)

The multiplication with eat is transformed into a shift on the s-axis (see Table 1.2)

8. Change of scale

The Laplace transform of xat for a real is:

Lxat=1aXsa	(1.51)

9. Convolution integral

The Laplace transform of the convolution integral over a time integral t of functions xt and gt is:

Lx(t)g(t−t)=Lg(t)x(t−t)=Lx∗g(t)=Gs⋅Xs.	(1.52)

10. Initial-value theorem

If the Laplace transform of x(t) is Xs then:

limt→0xt=lims→¥Xs.	(1.53)

The initial-value theorem computes the valuex0+ from the transform Xs. It is to be noted that x0+ is the initial value only for continuous functions xt. Otherwise, it represents the value of the function at a (time) value slightly greater than zero.

11. Final-value theorem

If the Laplace transform of xt is Xs and sXs is analytic on the imaginary axis and in the right half of the s-plane, then:

limt→¥xt=lims→0sXs.	(1.54)

The final value theorem gives the value ofxt when t→¥ directly from the Laplace transform X(s). For dynamic control systems it is very useful in the analysis of the steady-state response, since it gives the final value of the time function as the system response.

However, the final-value theorem is not valid if sXs contains any poles (zeroes of denominator) whose real part is zero or positive, which is equivalent to the analytic requirements of sXs stated in the theorem. So, one must take care in applying the final-value theorem.