### Continuous models with dead-time

Many real systems present an unavoidable time delay in the signal flow between the components. The delay usually results from the physical separation of the components and due to limited speed of material or energy transfer that occurs between that components.

This may be encountered in various types of systems, especially systems with hydraulically, pneumatically or mechanical transmissions (ex. pipes, transportation bands), thermodynamic systems, etc. Typically, for control systems this pure time delay appears as a delay between a change in the manipulated variable and its effect on the controlled plant, or as a delay in the measurement of the output signal. Such time delays appear in digital controllers too, due to their sequential operation and limited computing speed.

In literature this type of time delay is referred as dead-time or transportation lag, in order to distinguish it from the delays (time constants) associated with the speed of energy changes that occur in that dynamic system.

For simplicity and better understanding let consider as example a fluid with a temperature q flowing through a pipe. This is shown in Figure 2. The fluid velocity is u and it is considered constant with respect to time. The pipe length is L, so it takes a time Td=L/u for the fluid to move from one end to the other.

v

L

q1(t)

q2(t)

Figure 2. Process with dead-time (fluid flow trough a pipe)

Let q1(t) denote the incoming fluid temperature and q2(t) denote the temperature of the fluid living the pipe. Now, suppose that the temperature of the incoming fluid suddenly increases. If this is modelled as a step function, the result can be followed in Figure 3.

Td

t

q2(t)

t

q1(t)

Figure 3. Time plots of the input and the response (fluid flow trough a pipe)

In assumption that no heat energy is lost, q2(t), the temperature at the output, is q1(t−Td), where q1(t) is the temperature at the input. Thus a time Td later the output temperature suddenly increases with an equal value. A similar effect occurs for any change in q1(t), so in general we may write for our example:

q2(t)=q1(t−Td).	(1.86)

Applying the Laplace transform together with the time-shifting theorem, results:

Q2(s)=e−TdsQ1(s),	(1.87)

where Q1(s) and Q2(s) represent the Laplace transforms of variables q1(t) and q2(t), respectively.

If we generalise the above conclusions for models of linear continuous-time dynamic systems, the dead-time appears as a corresponding valued time-shift (delay) of the output variable. If we continue to note the time delay with Td, the general mathematical model is:

dny(t)dtn+a1dn−1y(t)dtn−1+K+an−1dy(t)dt+any(t)=b0dmu(t−Td)dtm+b1dm−1u(t−Td)dtm−1+K+bm−1du(t−Td)dt+bmu(t−Td)	(1.88)

Solving this model in time-domain means nothing else but solving it without dead-time and then shifting the result in time with a value equalling the dead-time.

Applying the Laplace transform to Equation (1.90) with zero initial conditions in order to determine the corresponding transfer function, we obtain:

DG(s)=Y(s)U(s)=(b0sm+b1sm−1+K+bm−1s+bm)e−Tdssn+a1sn−1+K+an−1s+an=G(s)e−Tds.	(1.89)

Hence, the transfer function of a system that includes a deadtime element equals the transfer function of the system without dead-time multiplied with the term e−Tds.

The presence of the dead-time means that the system does not have a characteristic equation of finite order. In fact our simple example shows that there are an infinite number of characteristic roots (zeros or poles) for a system with dead-time.This can be seen by noting that the term e−Tds can be expanded in an infinite Taylor-series as:

e−Tds=1eTds=11+Tds+Td2s22!+K .	(1.90)

In engineering practise one can use different polynomial approximations in algebraic modelling of dead-time systems. Two of them are commonly used; one rough:

e−Tds=1eTds@11+Tds .	(1.91.a)

and the other, more precise but increasing the computational effort:

e−Tds@1−Td2s1+Td2s .	(1.91.b)

e−Tds@1−Td2s+112(sTd)21+Td2s+112(sTd)2 .	(1.91.c)

The last two approximations are known as first-order Padè approximation and as second-order Padè approximation, respectively.

There could be (or already were) developed approximations of higher order, but nevertheless the enumerated are commonly used in practice.