### Description of discrete-time models using the z transform

The impulse representation of sampled signals provides a convenient way of introducing the z transform in describing the discrete-time signals, functions and models.

#### The z transform

Using Laplace transforms for mathematical description of discrete-time signals, we obtain Equation (1.95) as representation in the complex domain. To simplify much more the description and to obtain an algebraic representation of uniform sampled signals, the z transform is introduced by definition as a simply change in the complex variable:

z=esT0,	(1.157)

where s is the Laplace transform variable and T0 is the sampling period. Equation (1.157) also gives the inverse relation:

s=1T0lnz .	(1.158)

The z transform of an impulse train x(t) given by Equation (1.94) will be:

Zx(t)=X(z)=Xs=1T0lnz=i=0¥x(iT0)z−i=x(0)+x(T0)z−1+x(2T0)z−2+K=x0+x1z−1+x2z−2+K.	(1.159)

We have noted with X(z) the z transform of the discrete-time function x(t). The transformation led to a power series in z−1. Comparison of Equation (1.159) to Equation (1.94) shows that z−1 may be seen as a „time-shift operator“ equalling to a time delay T0 . The coefficients in the obtained power series are the values of the continuous function at the sampling instants. The bi-univoc correspondence between the amplitude modulated impulse train x(t) and its z transform is immediately seen.

The z transform is the discrete-time analogy of the Laplace transform for continuous systems. It is a convenient tool to study linear difference equations (discrete-time models) with or without initial conditions. The z transform maps a sequence of semi-infinite time sequence into a function of a complex variable. Notice the difference in range for the z transform and the time-shift operator calculus. The variable z is a complex variable and should be distinguished from the time-shift operator q, even if appearance suggests it.

The definition given by Equation (1.159) however, makes sense if X(z) converges. If x(iT0) is limited, then X(z) converges for z<1. This assumption can be made in the case of most discrete-time functions. The assumptions made at the Laplace transform, especially that x(kT0)=0 for k<0, are also valid for the z transform.

Illustrative example

Find the z transform of the sequence:

x=1,a,a2,a3,K with x<0 for t<0.

Solution:

The sequence values can be considered as coefficients of the transform’s series expression, that is:

X(z)=1+az−1+a2z−2+a3z−3+K=i=0¥aiz−i.

This is a geometric series that converges if z>a. (z=a is said to be the radius of absolute convergence). The sum of the geometric series can be expressed as:

X(z)=11−az−1=zz−a.

If a=1 the preceding sequence corresponds to the sample sequence of a unit step function that starts just before the sampling begins. The z transform of this function is:

Z1(t)=11−z−1=zz−1.

If a=exp(−bT0), the original sequence can be considered as samples of an exponential function of form: x(t)=e−bt. The z transform is obtained similarly and yields:

Z(e−bt)=11−e−bT0z−1=zz−e−bT0.

In general, the z transforms of more complex discrete-time functions are obtained by use of the same procedure as described in the preceding example. If a continuous time function x(t) is given as the starting point, the procedure of finding its z transform is to first form the impulse sequence x(t) and then use Equation (1.159) to get X(z). An equivalent interpretation of this step is to send the signal x(t) through an ideal sampler whose output is the impulse train x(t), then take the Laplace transform of x(t) to give X(s) and then obtain X(z) by substituting into Equation (1.157) or Equation (1.158).

Table 1.3 gives the z transforms of some commonly used (discrete) time functions. More extensive tables can be found in the literature.

Table 1.3. z transform of some (discrete-time) functions

| ftxt | xkfork³0 | FsXz |
| --- | --- | --- |
| dt- unitary impulse | 1 for k=0 | 1 |
| 1t- unitary step | 1 for k³0 | zz−1 |
| tnn! | knn! | lima→0(−1)nn!dndanzz−e−aT0 |
| at | ak | zz−a |
| tat | kak | az(z−a)2 |
| sinwt | sinwk | zsinwT0z2−2zcoswT0+1 |
| coswt | coswk | z(z-coswT0)z2−2zcoswT0+1 |
| atsinwt | aksinwk | azsinwT0z2−2azcoswT0+a2 |
| atcoswt | akcoswk | z(z-acoswT0)z2−2azcoswT0+a2 |