#### The inverse z transform

As in the case of Laplace transform, one of the major objectives of the z transform is, that for discrete-time signals (or functions, or models) algebraic manipulations can be made in the z domain and then the final time behaviour is determined by inverse z transformation. In general, the inverse z transform of the complex function X(z) yields information only on x(kT0), and not on x(t). In other words, the z transform carries information only in a discrete fashion, information on the signals only at the sampling instants. With this in mind, the inverse z transform can be performed by using one of the following three methods:

a) The inversion formula

The time sequence x(kT0) can be determined from X(z) with the inversion formula:

x(kT0)=12pjX(z)zk−1dz ,	(1.160)

that is a contour integration along the path G, where G is any closed path (for example a circle centred at the origin) in the z-plane that encloses all singularities of X(z)zk−1.

One way of evaluation of integral from Equation (1.160) is by use of the residue theorem of complex variable theory. Equation (1.160) is equivalent to writing:

x(kT0)=12pjX(z)zk−1dz=ReziduesofX(z)zk−1atpolesofX(z)zk−1.	(1.161)

For simple poles, the residue of X(z)zk−1 at the pole z=zi is obtained as:

RezofX(z)zk−1atpolezi=limz→zi(z−zi)X(z)zk−1.	(1.162)

For a multiple-order q pole z=zi, the residue of X(z)zk−1 at the pole z=zi is:

RezofX(z)zk−1atpolezi=1(q−1)!limz→zidq−1dzq−1(z−zi)qX(z)zk−1.	(1.163)

Illustrative example

Given the z transform functions:

a) X(z)=(1−e−aT0)z(z−1)(z−e−aT0) and b) X(z)=T0z(z−1)2

find the inverse z transforms by using the inversion formula (residue theorem).

Solution:

a) The given function has two poles: z1=1 and z2=e−aT0. Formula of Equation (1.162) gives:

x(kT0)=ResofX(z)zk−1atz1=1+ResofX(z)zk−1atz2=e−aT0==(1−e−aT0)zkz−e−aT0|z=1+(1−e−aT0)zkz−1|z=e−aT0==1−e−akT0.

b) The given function has a second order pole: z=1. Applying formula (1.163) we get:

x(kT0)=limz→1ddzT0zk=kT0zk−1|z=1=kT0 .

b) The power series method

In this case, the z transform X(z) is expanded into a power series in powers of z−1. In view of Equation (1.159) the coefficient of z−k is the value of x(kT0). The method can be easily programmed on a computer.

Illustrative example

Given the z transform function from the previous example a), find the inverse z transform by using the power series method.

Solution:

The given function can be expanded into a power series in powers of z−1 by long division:

X(z)=(1−e−aT0)z(z−1)(z−e−aT0)==(1−e−aT0)z−1+(1−e−2aT0)z−2+K+(1−e−akT0)z−k+K==i=0¥(1−e−aiT0)z−i.

Thus

x(kT0)=1−e−akT0 ,

which is the same result as was obtained before.

b) The partial-fraction expansion method

The z transform function X(z)is expanded by partial fraction expansion into a sum of simple recognisable terms, and the z transform table is used to determine the corresponding x(kT0). In carrying out the partial fraction expansion, there is a slight difference between the z transform and the Laplace transform procedures. With reference to the z transform table (Table 1.3), we have to note that practically all the transform functions have the term z in the numerator. Therefor, we should expand X(z)/z analogous to the partial fraction expansion presented in the case of Laplace transform and then multiply z across to obtain the desired form (for simple zeros), that is:

X(z)=K1zz−z1+K2zz−z2+K .	(1.164)

Illustrative example

Given the z transform function from the previous example:

X(z)=(1−e−aT0)z(z−1)(z−e−aT0) ,

find the inverse z transform by using the partial fraction expansion method.

Solution:

Expanding X(z)/z by partial fraction expansion, we have:

X(z)z=1−e−aT0(z−1)(z−e−aT0)=1z−1−1z−e−aT0.

Thus

X(z)=(1−e−aT0)z(z−1)(z−e−aT0)=zz−1−zz−e−aT0.

From the z transform table of Table 1.3., the corresponding inverse z transform is found to be:

x(kT0)=1−e−akT0 ,

which is the same result as was obtained in both previous examples.

#### Important theorems (properties) of the z transform

The following basic theorems extend the usefulness of the z transform method. They help one to obtain a better understanding of z transformations. The theorems are presented shortly, without proofing them.

1. Linearity of the z transform

The z transformation is a linear operation; that is, for any functions x1t and x2t whose z transform exists and for any constants c1 and c2:

Zc1x1t+c2x2t=c1Zx1t+c2Zx2t=c1X1z+c2X2z.	(1.165)

2. Real translation (z transform of a time-shifted function)

The real translation makes sense only if the time shift is an integer multiple of the sampling period. We will consider the time-shift equalling to nT0, with n integer.

a)Real translation to the right (time delay)

The z transform of a time-delayed discrete-time function xt−nT0 is equal to the z transform of xt multiplied by z−n, that is:

Zxt−nT0=z−nXz	(1.166)

The property is also known as shift into the past.

b)Real translation to the left

The z transform of a discrete-time function shifted to the left xt+nT0 is:

Zxt+nT0=znXz−i=0n−1x(iT0)z−i .	(1.167)

The property is also known as shift into the future. By shifting to the left, the function values of the original non-shifted function xt for i=0,1,...,n-1 vanish, because the z transform is defined only for positive time sequences.

3. Multiplication by t

The z transform of a time-function xt multiplied by its variable t and discretized in time with a sampling period T0, is:

Ztxt=-zT0ddzXz .	(1.168)

4. Multiplication by at/T0

The z transform of the dicretized time-function at/T0xt is:

Zat/T0x(t)=Xza .	(1.169)

5. Multiplication by eat

The z transform of the discretized time-function e−atxt is:

Ze−atx(t)=XzeaT0 .	(1.170)

Notice the change of sign of the coefficient a, or the effect of an exponential damping.

6. Convolution sum

The z transform of the convolution sum over n sampling periods of the discrete-time functions xkT0 and gkT0 is:

Z(x∗g)(t)=Zi=0nxiT0g(n−i)T0=XzGz=GzXz.	(1.171)

7. Initial-value theorem

If the z transform of xt is X(z) then:

limt→0xt=x(0)=limz→¥Xz.	(1.172)

8. Final-value theorem

If the z transform of xt is X(z), then the area of the impulse xkT0 as k→¥ is:

limt→¥xt=limk→¥xkT0=x(¥)=limz→1z−1zXz=limz→1(1−z−1)Xz.	(1.173)

#### The pulse transfer function and the z transfer function

The mathematical model of a system represents the relationships that exist between the input signal u(t) and the output signal y(t) of that particular system. If the considered signals are sampled (time-discretized) by an ideal sampler, the mathematical description of signals is that of infinite impulse trains as was established in Equation (1.94).

The sampler operates at the input of the system as a forcing element that generates periodical impulses. If we consider the time response (the solution of mathematical model) for a singular unitary impulse denoted by the weighting function g(t), the sampler at the output of the system will transform it to the impulse train:

g(t)=k=o¥g(kT0)d(t−kT0).	(1.174)

The impulse train approximation of the input signal is:

u(t)=k=o¥u(kT0)d(t−kT0).	(1.175)

The approximate impulse train of the output signal results as a convolution sum (the considered system is linear) over the infinite time interval:

y(t)=ν=o¥u(νT0)g(t−νT0)=ν=o¥u(t−νT0)g(νT0).	(1.176)

This means that for a specific time instant t=kT0 one obtains:

y(kT0)=ν=o¥uνT0g(k−ν)T0=ν=o¥u(k−ν)T0g(νT0).	(1.177)

As for the convolution integral for continuous-time systems, the value of the output signal at the time instant t=kT0 is given by the product sum of u(νT0) and the opposite time-shifted g(k−ν)T0. The time-shift can be realised either in the impulse response (the weighting function) or in the input signal, as given in Equation (1.176) and/or (1.177).