MODIFY Tempus S_JEP No. 07759-94	System Modeling project

1.5 System Identification

Compiled by:

Tibor Chován 1

Teresa Escobet 2

Joseba Quevedo 2

1 University of Veszprém, Department of Chemical Engineering Cybernetics,      Veszprém, Hungary

2 Universitat Politècnica de Catalunya, Department of Automatic Control and Computer Engineering, Terrassa, Spain

August 1995

Terrassa, Spain

# Grantholder's data:

name:	dr. Tibor Chován

home institution:	University of Veszprém, Department of Chemical Engineering Cybernetics, Hungary

address:	H-8201 Veszprém, POB 158, Hungary

telephone:	36-88-421709

fax:	36-88-426016

email:	chovan@kib.vein.hu

mobility grant no.:	6

mobility period:	18 June, 1995 - 12 August, 1995 (8 weeks)

host institution:	Universitat Politècnica de Catalunya, Department of Automatic Control and Computer Engineering, Terrassa, Spain

Tempus coordinator:	Professor Joseba Quevedo

address:	c/ Colom, 11., 08222 Terrassa, Spain

telephone:	(34-3) 7398144

fax:	(34-3 7398101

email:	joseba@esaii.upc.es

# Notes

·	The following notes were prepared for the System Identification lectures as a part of the System Modeling course material.

·	Our aim was to give students an insight to the basic methods of system identification and provide them a practical approach to the whole identification procedure.

·	Since lecture notes or other course material are not yet available at U.P.C., Terrassa, it had to be compiled during the mobility period. The work was based on the available literature and we utilized the experience of the host institute.

·	Although the compiled material reflects a revised structure and contains examples elaborated by the contributors, as a course material, it is basically a compilation of the respective and related paragraphs from the books and articles listed in the Reference.

·	In order to help the students in understanding the different concepts and methods a number of explanatory examples is included in the text. The example problems are ones commonly used in the literature. These examples were solved and the corresponding figures were generated using Matlab/Simulink including the Signal Processing Toolbox and the Identification Toolbox.

·	Since the previous chapters of the System Modeling course have not been available yet it is important to list the concepts and terms which are assumed to be covered in those chapters or in the earlier studies:

Systems,

Signals,

Classification of models,

Linear models,

Time domain models,

Stochastic models

Operator domain models,

Time delay operator,

Frequency domain models,

Sampling theorem, Aliasing

Correlation, Fourier transform,

Transformation of models,

Basic process dynamics,

Transfer functions,

ARX and ARMAX models,

·	The books and papers used in the text are listed in the References. The list of citations is given at each subtitles to help the following corrections. Some books and papers which are not cited directly in the text are marked by asterisk.

·	As the final index of the course note has not been available yet, the chapters and sections in this contribution are numbered from 1 and the titles are started at the first heading level.

# Content

1. Introduction	1

1.1 Approaches to model building	1

1.2 Application of a posteriori models	1

1.3 Classification of models	2

1.4 Classification of identification methods	3

2. Identification and experiments	6

2.1 General procedure of system identification	6

2.2 Preliminary experiments	8

2.3 Design of input signals	8

2.3.1 Sum of sinusoid signals	10

2.3.2 Filtered noise signals	11

2.3.3 Pseudo random binary sequence (PRBS)	11

2.4 Selection of sampling time and duration of experiments	12

2.5 Data treatment	13

3. Nonparametric identification methods	16

3.1 Transient response analysis	16

3.2 Frequency response analysis	19

3.3 Correlation analysis	22

3.4 Spectral analysis	24

4. Parameter estimation methods	27

4.1 Formulation of parameter estimation problem	27

4.2 Least squares estimation method (LS)	27

4.3 Generalized least squares method (GLS)	31

4.4 Instrumental variable method (IV)	32

4.5 Maximum likelihood identification (ML)	35

4.6 Prediction error methods	37

4.7 Recursive methods of parameter estimation	40

4.7.1 Recursive least squares method	40

4.7.2 Unified recursive parameter estimation algorithm	43

5. Model structure selection and model validation	45

5.1 General aspects of model structure selection	45

5.2 Preliminary data analysis for model structure selection	46

5.3 Criteria for comparing model structures	47

5.4 Model validation	49

References	51

# 1. Introduction

(compiled from: Fasol, 1980; Zhu, 1993; Johansson, 1993; Isermann, 1980)

In recent years the importance of system analysis and modeling has increased greatly in many sciences e.g. economics, biology, medicine (physiology), ecology and certainly in the field of process design and control. Especially in technical applications, system analysis and model building are the most important conditions for making the use of different design and control methodologies and theories. Applications of different kinds of system models in these fields require more and more accurate models of the system under consideration.

## 1.1 Approaches to model building (Fasol, 1980; Isermann, 1980)

Physical models are either reduced-size copies of the original system following the laws of model similarity, or analogies. The idea of an analogy implies that there exists "something" at every instant of time that is to be analogous to the dependent variables of the original physical system. Mathematical models, which are discussed in details in previous chapters, map the relationships between the physical variables in the system to be modeled onto mathematical structures like simple algebraic equations, differential equations or even systems of differential equations. Design of mathematical models is necessary to understand the dynamic behavior of industrial systems, and modeling enables a mathematical treatment of the systems.

Mathematical models can be developed in different ways: either purely theoretically based on the physical relationships which are a priori known about the system, or purely empirically by experiments on the already existing system, or by a sensible combination of both ways. Models obtained by the first method are often called a priori or first principle or theoretical models, while models obtained in the second way are called a posteriori or experimental (black-box) models. In case of theoretical analysis, the dynamic properties of the system are primarily taken care of by the respective balance equations. These balances are established by the laws of conservation supplemented with the necessary state-equations and phenomenological laws. Theoretical model building becomes unavoidable if experiments in the respective plant cannot or must not be carried out. If the plant to be modeled does not yet exist, theoretical modeling is the only possibility to obtain a mathematical model.

## 1.2 Application of a posteriori models (Johansson, 1993; Isermann, 1980)

In many cases the available knowledge is very limited and it can be very expensive to elaborate a priori models of the system. System or process identification is the modeling method based on experimental data. If experimental analysis of a process is performed, the input and output signals are measured. The measurements are then evaluated in an identification procedure yielding a mathematical model of the process. This identification enables model building for processes of arbitrary structure. Identical analysis methods can be applied to diverse, complicated processes. By measuring input and output signals one obtains a model for the input-output behavior of the process. The input signal can either be operating signals of the process (usually of stochastic nature), or artificial test signals. The result of the identification is an experimental model.

Utilizing modern computers, very efficient identification and parameter estimation methods have been developed. The mathematical models developed by these methods can be applied for a wide variety of diverse purposes:

·	Prediction or forecasting can serve to obtain better knowledge of the process, to verify theoretical models, to predict new phenomena, etc. In this context it is important to represent external actions and external disturbances and using knowledge of statistical characteristics of random variables, as there is usually little theoretical or practical possibility of determining such characteristics in advance.

·	Control system analysis and design provide a rich field for the application of modeling and identification since application of models allows quantitative predictions to be made concerning crucial features of control systems such as stability conditions and the development of oscillatory behavior.

·	Simulation based on mathematical models is widely used for the assessment of model complexity, for engineering design or for operator training, all of which require adequate modeling and adequate input.

·	Models often serve as the basis of monitoring or supervision, error detection and process diagnosis in large systems.

·	Industrial processes in continuos operation require system optimization for their economy, which in turn requires very accurate modeling.

·	Soft sensors can provide information on process variables which are directly cannot be measured or their measurement is troublesome or too costly. This variables can usually be computed from other measurement based on suitable models.

## 1.3 Classification of models (Fasol, 1980; Johansson, 1993)

Mathematical models were classified on a broad scale of aspects in the previous chapters. It is still practical to recall model classes which are the most important from the perspective of identification.

Mathematical models can be represented in time domain, in operator domain (e.g. Laplace, z, etc. transforms) or in frequency domain. Regarding time domain, the continuous time models are closer to physical considerations, whereas the discrete-time system behavior is considered to be defined at a sequence of time instants related to measurements. The discrete-time models are closely related to implementation problems of digital processing.

Many models have been defined by a given form but are dependent on a finite number of real parameters. Such models are said to be parametrized or parametric models, although there is no clear cut distinction from other models such as spectra or impulse responses, which are sometimes referred to as non-parametric models. Examples of parametric models are algebraic equations, differential equations, systems of these equations, transfer functions, etc. In these cases, the model building consists of the determination of parameters in fixed structures. The theoretical model building always yields a parametric model. A nonparametric model is the response obtained directly from an experimental analysis of a physical system. The recorded step response of a system, for example, is a nonparametric model. By various identification methods a parametric model can be produced from a nonparametric model. By experimental identification a parametric model can be obtained directly if the structure of the system can be defined a priori.

Very often - especially in control theory - systems even if they exhibit non-linear behavior and therefore the use of non-linear models would be appropriate, the system dynamics is described by linear models which allows simpler mathematical treatment. In this chapter methods for identifying linear models will be discussed. For methods of nonlinear identification please refer to Johansson (1993).

## 1.4 Classification of identification methods (Fasol, 1980; Isermann, 1980; Zhu, 1993)

A natural principle for classification of identification methods is the type of the resulting mathematical model. Here the practical implementation and the applied identification techniques are considered. First the following practical aspects of classification are reviewed:

(a)	errors between process and model;

(b)	algorithms used;

(c)	sequence of measurements and evaluation.

(a)	For judging the error between model and process one can use output-signal error or prediction errors. For mathematical reasons, generally those errors are preferred which depend linearly on the model parameters. Therefore, the output-signal error is utilized if the weighting function is used as a model, and prediction errors if difference or differential equations and transfer functions are used. To illustrate the difference between the two approaches let us consider the following ARMAX model:

µ §,	(1.1)

where µ §is the measured output of the process, µ §is the measured input while µ §is a non measured disturbance (noise). Then the prediction error (or equation error) is expressed as,

µ §,	(1.2)

while the output error utilizes the transfer function form for predicting the output in the following form:

µ §.	(1.3)

(b) Concerning the parameter identification methods one can distinguish between algorithms for direct estimation and algorithms for iterative estimation. With algorithms for direct estimation, the parameters are evaluated in a straightforward manner. On the other hand, algorithms for an iterative estimation evaluate the parameters stepwise. Furthermore algorithm can be recursive or non recursive. With recursive algorithms, the parameters are evaluated after each new set of data. The latest set of data is used to improve the parameter estimation from the proceeding step. With non recursive algorithms, the entire set of data is stored, and only one computation is used to find the parameters.

(c)	If a digital or process computer is used for the identification, two kind of coupling between the process and the computer have to be distinguished the off-line (indirect coupling) and the on-line (direct coupling). In off-line operation, first the data are stored in a data storage device. Later on the data is transferred to the computer and evaluated. For this type of identification mostly batch processing of data is applied, that means, the whole data set is evaluated at once. When running an on-line operation, the computer is directly coupled to the process, and directly fed with data. Then, two ways of processing can be distinguished: real-time processing when data is evaluated immediately after each sample or batch processing, when the data set is evaluated at once after the all measurements have been made. The various ways of processing the measured data are depicted on Figure 1.1.

µ §

Figure 1.1	Ways of processing for system identification (Isermann, 1980)

The most important classes of identification techniques suitable for digital computer implementation are summarized in the following:

(1)	Transient response analysis is mainly used for obtaining step and impulse responses. Respectively the test signals are usually impulse or step signals, and the recorded output constitutes the corresponding model.

(2)	Fourier analysis is mainly used for the determination of the frequency response characteristics of linear processes using continuous signals.

(3)	Correlation analysis is a time-domain method, and is suitable for linear processes with continuous as well as discrete signals. Admissible input signals are stochastic or periodic signals. As a result, correlation functions (and, a special case, the weighting function) are obtained.

(4)	Spectral analysis is used under the same conditions as the correlation analysis. The results are values of the frequency response characteristics.

The result of the Fourier, correlation and spectral analysis are nonparametric models. It is assumed that the processes are linearizable. A particular model structure does not have to be assumed. Therefore, those methods are suited for processes with an arbitrarily complicated structure.

If a parametric identification method is to be applied, a certain model structure, generally linear, has to be assumed. The model parameters are evaluated by minimizing an error between model and process.

(5)	The model-fitting methods are developed for models with continuous signals. They yield the parameters of differential equations. These methods will not be covered here.

(6)	The parameter estimation methods can be used with discrete as well as continuous signals. In most cases, the prediction error, and in some cases the output error are used as the error signal between model and process. If the error signal depends linearly on the parameters, simple estimation algorithms are obtained. For parametric identification methods, assuming the correct model structure is known in advance, an iterative search for the model order and dead time follows. The most important classes of parameter estimation methods are the methods based on the following principles: least square parameter estimation; maximum likelihood method.

The choice of identification method highly depends on the purpose of model building. A summary of the relationship between the methods and the goals is given in Table 1.1 (Isermann, 1980).

Table 1.1	Some examples for the relationships between the final goal of the model application and specifications of the process identification (Isermann, 1980)

| Final goal of model application | Type of process model | Required accuracy of model | Identification method |
| --- | --- | --- | --- |
| verification of theoretical models | linear, continuous-time, nonparametric / parametric | medium / high | off-line, transient response, frequency response, parameter-estimation |
| controller parameter tuning | linear, nonparametric, continuous-time | low for input / output behavior | off-line, transient response |
| computer aided design of digital control algorithms | linear, parametric, (nonparametric) discrete-time | medium for input / output behavior | on-line / off-line parameter- estimation |
| self-adaptive digital control | linear, parametric, discrete-time | medium for input/output behavior | on-line parameter-estimation |
| process parameter monitoring and failure detection | linear / nonlinear, parametric, continuous-time | high for process parameters | on-line parameter- estimation |

# 2. Identification and experiments

(compiled from: Fasol, 1980; Söderström, 1989; Landau, 1990; Zhu, 1993; Johansson, 1993; Isermann, 1980; Ljung, 1987)

As stated in the introduction, identification is a method for model building based on experiments. Zadeh defines identification as the determination on the basis of input and output, of a system (model) within a specified class of systems (models), to which the system under test is equivalent (in terms of a criterion). By this definition three basic entities are involved in system identification: the data, a set of models and a rule or criterion for model estimation. The data are usually obtained from suitably designed experiments. Then a suitable model is to be determined within set of candidate models. The next step is to find the best model from the data. Finally the model is tested to see whether it is an appropriate representation of the system.

## 2.1 General procedure of system identification (Söderström, 1989; Zhu, 1993; Isermann, 1980)

The overall scheme of the system identification procedure is given on Figure 2.1. In general terms, an identification experiment is performed by exciting the system (using some sort of input signals) and observing its input and output over a time interval. These signals are normally recorded in a computer mass storage for subsequent "information processing". It is very important to consider the application context, i.e. the intended purpose of modeling, whether the model derived is to be used in scientific research or engineering design (e.g. simulation models, fault detection models, control system analysis).

The design of experiments includes selection and determination of several factors, like input signals; sampling time; identification time; open or closed loop identification; off-line or on-line identification; equipment for the signal generation, data storage and computation; signal filtering; etc., taking into account constraints given by the operating conditions of the process, the properties of actuators and sensors.

The most theoretical part of the system identification procedure is the selection of model structure. It is here that a priori knowledge and engineering intuition and insight have to be combined with the formal (mathematical) properties of the models. The form of model, used in most of the following, is typically is a linear difference equation of a certain order.

The unknown parameters of the model are then determined using some statistically based parameter estimation method. To measure the goodness of the model fit a criterion (loss function) needs to be specified, Often the sum of squares of some error signals (residuals) is used as a criterion. In practice, the selection of structure and the estimation of parameters are often done iteratively. This means that a tentative structure is chosen first and the corresponding parameters are estimated. The main classes of identification methods which are mostly applied on the measured data are summarized in Table 2.1.

Model validation means the testing whether the estimated model is sufficiently good for the intended use of the model. First of all, it should be checked if the model is in agreement with the a priori knowledge of the system. Then it should be tested if the model can fit the experimental data well, preferably using a data sequence that has not been used in model estimation. The final validation of the model is the application of the model. If the model is inadequate for the given purpose the procedure is repeated in an iterative scheme, i.e. new experiments are conducted, some more complex model structure is considered, another method for parameter estimation is employed, etc.

µ §

Figure 2.1	General procedure of process identification (Söderström and Stoica, 1989)

Table 2.1	Main classes of identification methods (Isermann, 1980)

| Identification methods | Model |  |
| --- | --- | --- |
|  | nonparametric | parametric |
| step response techniques | + | + |
| frequency response techniques | + | + |
| Fourier- and spectral analysis | + |  |
| correlation techniques | + |  |
| parameter estimation |  | + |

## 2.2 Preliminary experiments (Zhu, 1993; Johansson, 1993)

Since some information is desirable to design efficient experiments it is natural and often necessary to make iterated experiments. A standard procedure of identification is to start with some preliminary (first stage) experiments then further experiments are conducted (second stage) for model estimation.

The preliminary experiments are aimed at the rough modeling of the process and can provide a basic qualitative understanding of the system. In general three types of simple experiments are considered.

(a)	In the free-run experiments none of the inputs are activated i.e. data is logged during normal operation. The intervention of the operator should be kept to the minimum. This type of experiments are directed to finding the characteristics of the disturbances on the process output.

(b)	The staircase experiments to test the range of linearity of the process, to finding the static gains and a rough estimation of the largest relevant time constant. Staircase test signals are applied to the selected candidate process inputs. The time interval of one stair should allow the process to reach its steady state. The steady state linearity of the process then can be checked by fitting a polynomial to the measured steady state responses. In case the higher order terms of the polynomial are significant, i.e. strong nonlinearity shows up, the inverse of the nonlinear model can be used as a precompensatore. If staircase experiments are not permitted for technical or economical reasons then simple steps, which are usually acceptable can be applied on the inputs. This still allows the estimation of the largest relevant time constant.

(c)	Finally, white noise experiments can be applied for determining the process bandwidth and time delays. In this type of experiments, the inputs are supplied with mutually independent white noise signals. The estimations are based on the spectral and correlation analysis of input and output signals. For details see Zhu and Backx (1993). The white noise signals can be approximated by a pseudo random binary sequence (PRBS) with a high clock frequency (PRBS is discussed in the next section).

## 2.3 Design of input signals (Söderström, 1989; Landau, 1990; Zhu, 1993; Isermann, 1980; Evans, 1992)

The process output variables are usually defined in the specification of the modeling problem, i.e. the variables to be controlled or predicted (simulated). Some considerations for selecting the input signals for control and simulation purposes are given in the following:

(1)	The selected inputs should have strong influence on the outputs both in amplitudes and in wide frequency range.

(2)	Reliable manipulation of the inputs should be available (for control purpose).

(3)	Input/output transfer have to be almost linear or linearizable (nonlinearities can be compensated e.g. using primary single loop controllers).

(4)	Measured but not manipulated inputs can be used as input signals in the identification process (this implies that the number of the inputs for identification can be larger than that for control).

(5)	The use of experience of the process operators and the available a priori knowledge of the process are essential for input selection. One of the purpose of the preliminary experiments is to support the selection of input signals.

The input signals used an identification experiment can have a significant effect on the resulting parameter estimates. The design of input signals involves both the selection of the form of the signal and the choice of its amplitude.

When choosing the amplitude of the test signal the following points should be taken into account:

·	There may be constraints on how much variation of the test signals is allowed during the experiments. For safety or economic reasons it may not be possible to introduce too large fluctuations in the process.

·	In practice most processes are nonlinear and linear models can hence only be an approximation. A linearization of the nonlinear dynamics will be valid only in some region. To estimate the parameters of the linearized model too large an amplitude of the input should not be chosen.

·	On the other hand an improvement in the accuracy of the estimates can be expected when the input amplitude is increased i.e. in statistical interpretation the attainable parameter accuracy µ § is inversely proportional to the µ §. This is natural since by increasing µ §, the signal-to-noise ratio will increase and the disturbances will play a smaller role. A good rule of thumb for a minimum amplitude to choose is that the effect of the input on the output in a diagram need at least be perceptible to the eye. Of course the correct choice of the amplitude is highly influenced by the frequency of the input signal.

In general, good parameter identification requires the application of a frequency-rich input signal. In order to guarantee that the estimation algorithm have unique solutions, some minimum requirement should be imposed on the test signals. This is called persistent excitation condition. A single variable discrete signal is said to be persistent exciting of order µ § if the following limit exists:

µ §	(2.1)

where the matrix

µ §

is positive definite and consequently it is invertible. It can be shown that a necessary condition for consistent estimation of a µ §th order linear process is that the test signal is persistently exciting of order µ §.

Besides the step function, which was discussed in 2.2, the following forms (types) of signals are the most frequently used ones in identification practice.

### 2.3.1. Sum of sinusoid signals

Sum of sinusoid signals is composed in the following way:

µ §	(2.2)

Where the amplitudes µ §, the frequencies µ § and the phases µ §have to be selected. It is well known that its spectrum has exactly µ § lines in µ §. Thus for the identification of an µ §th order process at least µ § sinusoids need to injected to the process. For good noise reduction the number of sinusoids should be much greater than the process order µ §. For selecting the phases an empirical method proposed by Schroeder can be applied:

µ §	(2.3)

Example 2.1

A multisine test signal, composed as a sum of four sinusoid signals is shown on Figure 2.2. The parameters corresponding to Eq (2.2) are µ §, µ § and µ §.

Figure 2.2	Sum of four sinusoid signals

### 2.3.2 Filtered white noise signals

Filtered white noise signals are usually based on a pseudo random number sequence and apply an autoregressive moving average process to generate the test signal, i.e.

µ §	(2.4)

The filter parameters should be chosen so that the polynomials µ § and µ § have all the zeros outside the unit circle. This type of signal has a continuous spectrum over the frequency range. Hence a filtered white noise is persistent exciting for any finite order.

### 2.3.3 Pseudo random binary sequence (PRBS)

Pseudo random binary sequence are sequences of rectangular pulses, modulated in width, that approximate a discrete-time white noise and thus have a spectral content rich in frequencies. They owe there name pseudo random to the fact that they are characterized by a sequence length within which the variations in pulse width vary randomly, but that over a large time horizon, they are periodic, the period being defined by the length of the sequence.

The PRBS can be generated by means of shift registers with feedback (implemented in hardware or software). The maximum length of a sequence is µ § in which µ § is the number of cells of the shift register. Figure 2.3 presents the generation of a PRBS of length µ § obtained by means of a 6-cells shift register. Note that at least one of the µ § cells of the shift register should have an initial logic value different from zero. Table 2.2 gives the structure enabling maximum length PRBS to be generated for different number of cells. A basic choice of the clock period of the PRBS generator µ § is to use the sampling time µ § of the system i.e. µ §. Note also a very important characteristic element of the PRBS: the maximum duration of a PRBS impulse is equal to µ § (number of cells).

In order to correctly identify the steady-state gain of the plant, the duration of at least one of the PRBS pulses (e.g., the maximum duration pulse) must be greater than the rise or settling time µ § of the plant, i.e. µ § since the maximum duration of a pulse is µ §. This condition can result in fairly large values of µ § corresponding to a sequence length of prohibitive duration. Therefore in a large number of practical situations a multiple of the sampling time µ § is chosen as the clock period for the PRBS generator that is more favorable than increasing the number of cells in the shift register. However, it is recommended to choose the multiplier equal to or less than 4.

Table 2.2	Generation of maximum length PRBS (Landau, 1990)

| Number of cells | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Sequence length | 3 | 7 | 15 | 31 | 63 | 127 | 255 | 511 | 1023 |
| Bits added | 1, 2 | 1, 3 | 3, 4 | 3, 5 | 5, 6 | 4, 7 | 2, 8 | 5, 9 | 7, 10 |

µ §

µ §

Figure 2.3	PRBS generation with a 6-cells shift register

(a) Scheme of the PRBS generator

(b) PRBS signal obtained (initially all zeros except B1=1)

## 2.4 Selection of sampling time and duration of experiments (Söderström, 1989; Landau, 1990; Zhu, 1993; Johansson, 1993; Isermann, 1980; Ljung, 1987)

For the identification of discrete time models the sampling time has to be selected properly before the experiments starts, because it cannot be decreased afterwards, if the sampled signals are stored. Of course sampling time can be doubled, tripled, etc. by deleting corresponding data. In computer data acquisition, it is unavoidable that sampling leads to information losses. Therefore, it is important to select the sampling frequency so that these losses are insignificant for the given application. In most of the cases the sampling is carried out with equidistant sampling instants. The sampling interval must be chosen by a trade-off and primarily the following considerations are to be taken:

·	In many cases the final application of the model defines the possible choices of sampling interval. Considering e.g. digital control the control equipment, the instrumentation, the type of control algorithm, etc. determine the available set of sampling period.

·	The information loss due to sampling is best described in frequency domain. It is well known that a sinusoid with a frequency higher than half of the sampling frequency (called Nyquist frequency) cannot, when sampled, be distinguished from one under this frequency. Consequently the part of the signal spectrum that corresponds to the higher frequencies will be interpreted as contributions from lower frequencies. This is the so called alias phenomenon. It also means that the spectrum of the sampled signal will be a superposition of different parts of the original spectrum.

The information about frequencies higher than the Nyquist one is thus lost by sampling. It is important not to make the bad worse by letting the aliasing effect distort the interesting part of the spectrum below the Nyquist frequency. This can be achieved by an analogous presampling filter which is also called antialias filter. The bandwidth of the filter should be somewhat smaller than half the sampling frequency. For high frequencies the gain should drop quickly. It is practical to apply the same filter on the output and the input signal as well. Beside decreasing the aliasing effect, the filter also increase the signal-to-noise ratio.

·	Assuming that the total time interval for an identification experiment is fixed it may be useful to sample the signals at a high sampling rate, since then more measurements from the system are collected.

·	If the total number of data points is fixed then the sampling interval must be chosen by a trade-off. If it is very large, the data will contain very little information about the high-frequency dynamics. If it is very small, the disturbances may have relatively large influence. Furthermore, in this last case, the sampled data may contain little information on the low-frequency dynamics of the system.

·	As a rule of thumb 5 to 15 - most frequently around 10 - samples per settling time µ § should be taken. It may often be much worse to select the sampling interval too large then too small.

·	Very short sampling intervals will often give practical problems: all poles cluster around 1 in the z-plane and model determination becomes a numerically very sensitive procedure. In many cases a fast sampled model will be nonminimum phase even if the original continuous-time process is minimum phase, and a process with time delays may be modeled with delays of many samples which can cause difficulties e.g. in controller design.

The duration of experiments also effects both the attainable parameter accuracy, which is reversely proportional to the experiment duration, and the spectral resolution. A rule of thumb is that the experiment duration need to be chosen greater then 5-10 times the longest time constant to be determined in the identification. Should the estimated variance not decrease as the experiment duration increases one might suspects that the parameter estimates are not statistically consistent. It should be kept in mind that one experiments seldom results in knowledge of the transfer function over a wider frequency range than two or three decades. Finally the experiment duration is also a trade-off between data economy and experiment economy.

Another aspect to consider when using a PRBS test signal is that in order to cover the entire frequency spectrum generated by a particular PRBS, the length of the test must be at least equal to the length of the sequence. In a large number of cases, the duration of the test is chosen equal to the length of the sequence.

## 2.5 Data pretreatment (Söderström, 1989; Zhu, 1993; Isermann, 1980; Ljung, 1987)

When data have been collected from the identification experiment, they are not likely to be in shape for immediate use in identification algorithms. There are several possible deficiencies in the data that should attended to:

·	High frequency disturbances in the data record,

·	Occasional burst and outliers,

·	Drifts, offset and low frequency disturbances,

·	Numerical values of different signals are not in the same order of magnitude,

·	Presence of significant time delays.

After the data collected, one should always first plot the data in order to inspect them for these deficiencies. Then the following treatments can be applied to avoid problems at the estimation steps later:

High frequency disturbances indicate that the choices of sampling interval and presampling filters were not thoughtful enough. If the sampling interval was unnecessarily short, the data may be resampled by picking every sth sample from the original record. Then, however, antialias filter must be applied before the resampling.

Burst and outliers in the measured data may have a very substantial influence on the resulting estimate. It is thus necessary to protect the estimate from bad data. it is usually much easier to detect bad data in a residual plot. Some signal processing steps are listed below to remove these kind of disturbances:

-	Clip the signal amplitudes to values never to be reached by the real process signals (using a priori knowledge);

-	Compute the standard deviation of the clipped signal;

-	Interpolate all samples of the original signal that are outside a band defined by the trend signal and by a multiple of the standard deviation.

An obviously erroneous measurement can also be changed "by hand" to an appropriately interpolated value. In advanced applications, failure-detection algorithms could be used to flag bad data. In every case, extreme care should be taken in order not to falsify the original data.

Slow disturbances (offset, trends, drifts) are not uncommon in measured data. They typically stem from external sources which are preferred not to be included in the model. There are basically two approaches to deal with such problems:

-	Removing the disturbances by explicit treatment of the data;

-	Letting the noise model take care of the disturbances.

The first approach involves removing the trends and offsets by direct subtraction. In practice the following techniques can be applied:

(1)	The steady-state levels of the signals (µ §, µ §) are determined at the physical equilibrium and then the deviation of the measured signals (µ §, µ §) from the equilibrium values is applied later on

µ §

µ §	(2.5)

This approach emphasizes the physical interpretation of the model as a linearization around the equilibrium.

(2)	The sample means are determined then subtracted according to Eq. (2.4)

µ §

µ §	(2.6)

This approach is closely related to the first one.

(3)	The offset can be explicitly estimated by extending the model and using the variables in the original physical units

µ §	(2.7)

The offset µ § then corresponds to and is contained in the parameter vector.

In case of trends or drifts straight lines or curve segments can be fitted to the data similar manner as the constant level (Eq. 2.6) and deviations from these time-varying means are considered.

The second approach basically means thatµ § is viewed as a constant disturbance and described as a part of the noise model. In the simplest case this basically equivalent to differencing the data and then using it in the original model:

µ §

µ §	(2.8)

This method will push the fit into a high-frequency region, which is unsuitable for many applications.

Scaling of the variables is desirable in many cases since in industrial practice not all input and output signals have the same order of magnitude. This problem is especially important in multivariable identification when evaluating the loss function. The problem may be overcome by correcting the signals first for the offset and by scaling them afterwards with respect to the known range of the instruments or the signals.

Many industrial and other processes have time delays that are not negligible. It is very important to describe the delay correctly in the model. In parameter estimation methods not considering delays explicitly, known delay times can be treated by shifting input and output relative to each other. One of the signals is used as reference and the remaining signals are shifted in time in order to compensate delays. Usually the input signal is shifted, e.g. in case of a time delay µ §

µ §	(2.9)

Using the shifted variables the parameters of the model can be estimated using the standard methods.

# 3. Nonparametric identification methods

(compiled from: Ljung, 1987; Söderström, 1989; Johansson, 1993; Rake, 1980; Godfrey, 1980)

A linear time-invariant system can be described by its transfer functions or by the corresponding impulse responses, etc. Here methods for determining these functions by direct techniques will be discussed. Such methods most often results in nonparametric models since they, in most of the cases, do not employ a finite-dimensional parameter vector in the search for the best description. There are methods in the literature (Schoukens and Pintelon, 1991) which provides parametric models from these nonparametric results. The following methods will be presented here:

·	Transient response analysis means that the input is taken as a certain nonparametric test signal, usually an impulse or step signal, and the recorded output constitutes the model. Some methods for obtaining parametric models directly from the transient response will also be discussed in this section.

·	Frequency response analysis employs a sinusoid test signal. For a linear system in steady state the output will also be sinusoidal. The change in amplitude and phase will give the frequency response for the used frequency.

·	Correlation analysis is based on a white noise input signal. A normalized cross-covariance function between output and input will provide an estimate of the weighting function.

·	Spectral analysis estimates the frequency response for an arbitrary input by dividing the cross-spectrum between output and input to the input spectrum.

## 3.1 Transient response analysis (Rake, 1980; Ljung, 1987; Söderström, 1989; Johansson, 1993)

Nonperiodic test signals in common use, are impulses of different shape and relatively short duration, step functions and ramp functions. A collection of nonperiodic test signals is shown on Figure 3.1. When choosing a test signal for a specific application the properties of the signal, the possibilities for generating and applying the signal and the kind of information wanted from the test are the main factors to be considered.

µ §

Figure 3.1	Typical nonperiodic test signals

The identification methodology based on the application of an impulse on the input of the system is known as impulse response analysis. If a system that is described in impulse transfer function form

µ §	(3.1)

is subjected to a pulse input

µ §	(3.2)

then the output will be

µ §	(3.3)

by definition of µ § and the impulse response µ §. If the noise level is low, it is thus possible to determine the impulse-response coefficients from an experiment with a pulse input. The estimates will be

µ §	(3.4)

and the error of the estimate is µ §. The weakness of the method that it is difficult to handle both low- and high-frequency properties even under noise-free conditions. In fact, the following practical problems of impulse response analysis can be listed:

·	restricted to stable systems,

·	difficulties in generating the input,

·	dynamics of sample and hold circuits,

·	synchronization between impulse and sampling,

·	difficulties with large input magnitudes,

·	saturations, nonlinearities,

·	difficulties with the "tails" due to their long duration and low amplitudes,

·	sensitivity to noise.

A complement to impulse response is step response analysis which is often easier to perform than impulse response analysis. The test signal is

µ §	(3.5)

Applying it on the system described by Eq. (3.1) the output is

µ §	(3.6)

From this the impulse response is obtained as

µ §	(3.7)

which has an error µ §. In most practical application using Eq. (3.7) results in large errors.

Example 3.1

The following second order system was studied applying impulse and step test signals:

µ §

The step signal of amplitude 1 was applied at µ § time. The sampling time was 1 sec. Figure 3.2 shows the original and the sampled signals of the step response and the impulse response obtained according to Eq. (3.7) as well as the continuous impulse response obtained by differentiating the original step response.

Figure 3.2	Transient responses of the system.

(a) Step response

(b) Impulse response

Very often it is of interest to fit a simple low-order model to a step response. The low-order model then can be used e.g. for controller design. These methods provide simple, direct estimations of parametric models. Almost all methods for the evaluation of step response functions use a small number of characteristic values of the response function. One of the oldest and best known method is that introduced by Küpfmüller giving a first order model with time delay which can be described by a first-order differential equation

µ §	(3.8)

or in transfer function form

µ §	(3.9)

The parameters µ §, µ § and µ § can be derived from the step response as indicated in Figure 3.3 with µ § being the amplitude of the input step signal.

Another method developed by Strejc gives estimates of parameter values for a model of order µ § with equal time constants and a time delay:

µ §	(3.10)

A further method is proposed by Schwarze which allows the evaluation of both step responses and ramp responses for several different model structures. For details on Strejc's and Schwarze's methods see Rake (1980).

µ §

Figure 3.3	Evaluation of step responses according to Küpfmüller

Example 3.2

Let us consider the same system as in Example 3.1. Based on the step response shown on Figure 3.2 and applying Küpfmüller's method the parameters of a first order model were determined. The parameters of the estimated model are µ §, µ § sec, µ § sec. The step responses of the original and the estimated models are compared in Figure 3.4. This example shows that although the estimates of the static gain and the delay are reasonably good, the simulated step response is not correct.

Figure 3.4	Comparison of step responses for the original and the estimated model

## 3.2 Frequency response analysis (Rake, 1980; Ljung, 1987; Söderström, 1989; Johansson, 1993)

The frequency response analysis is a simple method for obtaining detailed information about a linear system. The fundamental physical interpretation of the transfer function µ § is that the complex number µ § bears information about what happens to an input sinusoid. Thus applying the input signal

µ §	(3.11)

the response of the system can be described as

µ §	(3.12)

where

µ §	(3.13)

Assuming that we can ignore the transient from the initial conditions and only consider the input-output response, the stationary response of a stable system can be used directly to determine µ §. Simple observation of a sinusoidal input plotted against the corresponding output response allows direct calculation of the gain and the phase shifts. Figure 3.5 shows the evaluation of a sinusoidal response. The gain µ § is obtained as µ § while the phase µ § is µ §. Then the experiment is repeated for a number of frequencies in the interesting frequency band. Results can be presented in the form of a Bode diagram where the gain and the phase shift are plotted against the frequency in the specified range. The procedure outlined here is rather sensitive to disturbances and does not yield any statistical averaging over the measurement interval.

µ §

Figure 3.5	Evaluation of sinusoidal response

An improved method, called the correlation method of frequency response analysis reduces the effect of disturbance by applying time averages. The system output is multiplied with a sine and a cosine signal of the frequency of the system input respectively and then the two channels are integrated for a specified measurement interval µ §:

µ §	(3.14)

µ §	(3.15)

Estimates of the magnitude and the phase responses then can be expressed as:

µ §	(3.16)

µ §	(3.17)

The scheme of the correlation method of frequency response analysis is given on Figure 3.6.

µ §

Figure 3.6	Frequency response analysis with correlation technique

To minimize the effect of disturbance, the measurement duration µ § is always chosen as a multiple of full periods of the test signal. It can be shown that the noise is practically correlated out and therefore selecting a sufficiently long measurement interval the desired accuracy of the estimates can be reached even in case of low signal-to-noise ratio.

Equipment that performs frequency analysis based on the correlation method is commercially available. The main disadvantage of frequency response analysis is that many industrial processes do not admit sinusoidal inputs during normal operation. Repeating the experiments for many frequencies may lead to prohibitively long experimentation period.

Example 3.3

Frequency response analysis using correlation technique is conducted on the system of Example 3.1. The test signal is a sinusoid signal in the µ § frequency range. The output is disturbed by a white noise with µ §. An example of the test signal and the response at µ § is presented on Figure 3.7. The result of the frequency response analysis is presented in the form of the Bode diagram of the system on Figure 3.8. The solid lines represents the frequency response determined by correlation technique while the dotted lines are the real frequency responses. It can be concluded that the correlation technique provides a quiet good estimation of the frequency response. At higher frequencies the response is distorted which can be mainly attributed to numerical errors. Note that the phase response has to be corrected by µ § at higher frequencies according to Eq. (3.17).

µ §

Figure 3.7	Frequency response of the system at µ §.

## Figure 3.8	Bode diagram obtained by correlation technique.

(a) Magnitude response

(b) Phase response

## 3.3 Correlation analysis (Söderström, 1989; Godfrey, 1980)

Correlation analysis is a method based on statistical analysis for estimation of the impulse response function. The corresponding correlation function of the input and output signals are calculated to determine the impulse response. In most of the cases the input test signal is a white noise sequence or in practice a PRBS signal is used.

The model used in correlation analysis is an impulse response or weighting function of the following form

µ §	(3.18)

or its continuous time counterpart. Assume that the input is a stationary stochastic process which is independent of the disturbance. Then the equivalent relationship between the input-output crosscorrelation function and the input autocorrelation function (called the Wiener-Hopf equation) is

µ §	(3.19)

where the corresponding correlation functions are

µ §

µ §	(3.20)

where µ § means the expected value of a random variable. In practice the correlation functions are estimated from the data in the following form

µ §

µ §	(3.21)

Then an estimate µ § of the weighting function µ § can be determined in principle by solving

µ §	(3.22)

This will give a linear system of infinite dimension. The problem is greatly simplified if special input test signals with simple autocorrelation functions are applied. If a white noise test signal is used as an input then it is a priori known that µ § for µ §. For this case Eq. (3.19) reduces to

µ §	(3.23)

which can be easily evaluated from experimental data using Eq. (3.21). Equipment exists for performing correlation analysis automatically in this way.

Example 3.4

Correlation analysis was used to determine the impulse response of the system described in Example 3.1. The test signal is a white noise signal with µ §. The resulting impulse response is shown on Figure 3.9. For comparison the continuous impulse response of the system (determined in Example 3.1) is plotted also. It can be seen that the correlation analysis provides a reasonable estimate of the impulse response from a relatively simple experiment.

µ §

Figure 3.9	Impulse response determined by correlation analysis.

Another approach to the simplification of Eq. (3.22) is to consider a truncated weighting function. This will lead to a linear system of finite order, assuming that

µ §	(3.24)

Such a model is often called a finite impulse response (FIR) model in signal processing applications. The integer µ § should be chosen to be large in comparison with the dominant time constant of the system. Then the reduced form obtained from Eq. (3.22) using Eq. (3.24) will be a good approximation:

µ §	(3.25)

Writing out this equation for µ § , the following linear system of equations is obtained:

µ §	(3.26)

The above equation can also be applied with more then µ § different values of µ § resulting in an overdetermined linear system of equations.

## 3.4 Spectral analysis (Söderström, 1989; Godfrey, 1980; Ljung, 1987; Johansson, 1993)

Spectral analysis for determining transfer functions of linear systems was developed from statistical methods for spectral estimation. It is one of the powerful tools for analysis of experimental data by means of signal processing. One of the basic methods for spectral analysis is a direct approach via the discrete Fourier transforms.

Starting with the model described by Eq. (3.18) the following relation, corresponding to Eq. (3.19), can be derived if the input signal is stationary and it is uncorrelated with the noise:

µ §	(3.27)

where the cross-spectral density function µ § and the power spectral density function µ § are obtained by the Fourier transform of the corresponding correlation functions:

µ §	(3.28)

µ §	(3.29)

Note that µ § is complex valued and the physical interpretation of µ § is the energy density function since it can be shown that µ §. Then the transfer function can be estimated as

µ §	(3.30)

The next step is to find a reasonable method for estimating the spectral densities. Introducing the estimate of the crosscorrelation function given by Eq. (3.21) into Eq. (3.28) the cross-spectral density function is estimated as

µ §	(3.31)

Then making the substitutions µ § the following result is obtained

µ §	(3.32)

which can be written in a concise form

µ §	(3.33)

where

µ §

µ §	(3.34)

are the discrete Fourier transforms of sequences µ § and µ §. For µ § they can be computed efficiently using fast Fourier transform algorithms (FFT).

In a similar way

µ §	(3.35)

Then employing Eq. (3.33) and Eq. (3.35) in Eq. (3.30) the estimation of the transfer function can be expressed as

µ §	(3.36)

This quantity is sometimes called the empirical transfer function estimate.

Example 3.5

The system described in Example 3.1. was studied by spectral analysis in order to obtain the transfer function. The test signal is a white noise signal with µ §. The estimated transfer function is presented in the form of a Bode plot on Figure 3.10. The solid lines show the spectral analysis estimates while the dotted lines represent the true frequency responses. The empirical transfer function estimate has a significant error at higher frequencies both in the magnitude and in the phase plots. Although the method provides less accurate results than the frequency analysis by correlation method (Example 3.3) still it gives a reasonable first estimate of the transfer function in a simple experiment with white noise and in much shorter time than the frequency analysis by correlation method.

## Figure 3.10	Bode diagram obtained by spectral analysis.

(a) Magnitude response

(b) Phase response

For obvious reasons, the measurements is made on a finite-time interval only. Since the methods are based on a Fourier transform theory with infinite measurements sequences, the use of estimates based on truncated measurements may result in a systematic distortion of the calculated spectra and therefore of the estimated transfer function. A remedy to the problem is to introduce appropriate window functions which are some weight functions applied to data in order to reduce inaccuracies associated with the finite observation interval. For details see Johansson (1993).

# 4. Parameter estimation methods

(compiled from: Söderström, 1989; Johansson, 1993; Godfrey, 1986; Zhu, 1993; Strejc, 1980; Åström, 1971; Åström, 1980; Ljung, 1987; Isermann, 1991)

## 4.1 Formulation of parameter estimation problem (Godfrey, 1986; Strejc, 1980)

As an introductory problem, let us consider the following system, described by its discrete transfer function. The measurable input sequence is µ §, while the measurable output sequence is µ § which is corrupted by measurement noise µ §:

µ §	(4.1)

or in the usual time domain form:

µ §

The structure of this simple system is depicted on Figure 4.1 for the sake of clarity. Based on the measured input and output data the task is to determine an estimate of µ § and µ § polynomials satisfying some criterion for the model fit.

µ §

Figure 4.1	Structure of the model

## 4.2 Least squares estimation method (LS) (Söderström, 1989; Johansson, 1993; Godfrey, 1986; Strejc, 1980)

In general the parameter estimation problem of linear parameter models can be treated as a linear regression problem. The linear regression is the simplest type of parametric models. Its origin can be traced back to Gauss (1809), who used such a technique for calculating orbits of the planets. The corresponding process structure can be rewritten in the following form:

µ §	(4.2)

where µ § is a measurable quantity, µ § is an n-vector of known quantities, µ § is an n-vector of unknown parameters, i.e. the parameter vector, while µ § is the process noise. In case of our original model, Eq. (4.1), the corresponding elements are defined as µ § and µ §. The variable µ § takes integer values. It is straightforward to extend this model to the multivariable case.

The problem is to find an estimate µ § of the parameter vector µ § from the measurements µ §, µ §, ..., µ §, µ §. Given these measurements, a system of linear equations is obtained, namely

µ §

This can be rewritten in matrix notation as

µ §	(4.3)

where µ § and µ § are µ § vectors,

while µ §, is an µ § matrix.

One way to find µ § from Eq. (4.3) would be of course to choose the number of measurements, µ § to be equal to µ §. Then µ § becomes a square matrix. If this matrix is nonsingular the linear system of equations could be solved for µ § (not considering the noise term). In practice, however noise, disturbances and model misfit are good reasons for using a number of data greater than µ §. With the additional data it should be possible to get an improved estimate. When µ § the linear system of equations, (4.3), becomes overdetermined. An exact solution will then in general not exist.

Now let us introduce the equation errors as

µ §	(4.4)

or in vector form

µ §	(4.5)

The error generation is illustrated by Figure 4.2.

µ §

Figure 4.2	Generation of equation error

In statistical literature the equation errors are often called residuals. The least squares estimate of µ § is defined as the vector µ § that minimizes the loss function

µ §	(4.6)

where µ § denotes the Euclidean vector norm.

According to Eq. (4.4), the equation error µ § is a linear function of the parameter vector µ §. The corresponding minimum value of µ § can be obtained by taking the gradient of the optimization criterion

µ §	(4.7)

then

µ §	(4.8)

where the minimum, µ §, provides the following equation,

µ §	(4.9)

after a rearrangement of Eq. (4.9) we obtain:

µ §	(4.10)

The least squares estimate then can be expressed as

µ §	(4.11)

It can be shown that Eq. (4.9) is the necessary condition for obtaining a minimum and it is also a sufficient condition if µ § is invertible.

Least squares optimality has several attractive features for purposes of identification. First, large errors are heavily penalized. Second

, least squares estimates can be obtained by straightforward matrix algebra. Third, the least squares criterion is related to statistical variance and the properties of the solution can be analyzed according to statistical criteria. Assuming that the noise components are uncorrelated with the regressors and that µ § and µ § (white noise), the least squares estimates has the following statistical properties:

(a)	µ § is an unbiased estimate of µ §. It is easy to show that

µ §,

and then it follows from the assumptions that

µ §.

(b)	The covariance matrix of µ § is

µ §

(c)	An unbiased estimate of µ § is	µ §

Proofs for statement (b) and (c) can be obtained in a straightforward way, according to the assumptions above.

Example 4.1

The following system was studied (the parameters are defined in Table 4.1):

µ §

A model of the same order was to be determined by least squares estimation. The test signal, applied on the input was a PRBS. The input and output data were collected for two independent experiments for 200 sampling periods. In the first experiment µ §, while in the second experiment a coloured noise of µ § was applied, where µ § is a white noise with a variance of 0.9. The corresponding element of the least squares estimation are:

µ § and µ §

The input signal and the residual values in the two experiments are given on Figure 4.3. The estimated and real values of the parameters are summarized in Table 4.1. It is concluded that both the residual values and the estimated parameters demonstrate that the quality of estimation is quite poor in case of coloured noise.

Table 4.1	Estimated parameters in case of white and in case of coloured noise

| Parameter | Real value | Estimated value |  |
| --- | --- | --- | --- |
|  |  | White noise | Coloured noise |
| µ § | -1.5 | -1.4505 | -1.3191 |
| µ § | 0.7 | 0.6534 | 0.5264 |
| µ § | 1.0 | 1.0128 | 1.0618 |
| µ § | 0.5 | 0.5489 | 0.6641 |

Figure 4.3	Comparison of the residuals for white noise and coloured noise experiments.

(a) White noise experiment.

(b) Coloured noise experiment.

(c) Input signal.

## 4.3 Generalized least squares method (GLS) (Åström, 1971; Zhu, 1993)

As it was shown the least squares method can be expected to give a consistent (unbiased) estimate if the noise is uncorrelated and has zero mean. A possible way to overcome the difficulties caused by the correlated noise is the application of the generalized least squares method which is an extension of the ordinary least squares principle.

Consider the basic problem described by Eq. (4.1) and assume that the noise is correlated and can be expressed with white noise(µ §) in the following form:

µ §	(4.12)

and then the original process takes the following form:

µ §	(4.13)

If the discrete transfer function of the noise, µ §is known then the model can be transformed as

µ §	(4.14)

where

µ §

µ §

Hence if signals µ § and µ § are considered as the input and output signals we have an ordinary least square problem. Therefore the generalized least squares can be interpreted as a least squares identification problem where criterion is the generalized error, i.e.:

µ §	(4.15)

This solution is also referred as using "whitening filters" and depicted on Figure 4.4.

µ §

Figure 4.4	Error generation of generalized least squares

The correlation of the noise and the discrete transfer function µ § are seldom known in practice. To overcome this problem a multistep iterative procedure can be applied which estimates a noise model based on the residuals and then estimates the process model based the filtered variables. The procedure consists of the following steps:

(1)	Make an ordinary least squares estimation using the original set of data. This step gives a possible biased estimate of µ § and µ §:

µ §	(4.16)

(2)	Analyze the residuals of µ § and fit an autoregressive model to approximateµ §:

µ §

µ §	(4.17)

The estimation of µ § requires again an ordinary least squares estimation.

(3)	Filter the process input and outputs using the noise model obtained above (whitening):

µ §

µ §	(4.18)

(4)	Make a new least squares estimation using the filtered input and output signals (µ § and µ §). Repeat the procedure from the second step until convergence reached.

For checking the convergence simulation methods or test of the loss functions can be used. Since the GLS method aims at obtaining white noise residuals a natural way which can be used both for model validation and order selection is to check the whiteness of the residuals. The simple autocorrelation function can be used for this test. Note that here both the process order and the order of the disturbance filter have to be selected. To simplify the procedure both orders can be the same.

There are several ways to modify the GLS algorithm in order to simplify the computation or to speed up the convergence rate. The main idea of these modifications is first to apply the ordinary LS method on the transformed model (Eq. 4.14) with the originally measured variables in order to get an estimate of µ § and µ §, then perform some kind of model reduction to retrieve some reasonable estimate for µ §, µ § and µ § (for details see Söderström and Stoica, 1989).

## 4.4 Instrumental variable method (IV) (Söderström, 1989; 1986; Zhu, 1993; Strejc, 1980; Ljung, 1987; Åström, 1971)

Generalized least squares method, which was discussed above, gives a model of the environment in terms of a noise model as a filter driven by white noise. Another method to overcome the problems emerging from the presence of correlated noise is the instrumental variable method, which estimates the system dynamics without providing a noise model. The basic idea of the method is to modify the model equation by introducing so called instruments in order to cancel the effect of correlated noise.

Recall the process given by Eq. (4.2) which was used in introducing the least squares estimation method and assume that µ § is a signal vector with the same dimension as µ §. The entries of µ § are signals to be chosen in such a way that they are correlated with the input signal µ § but uncorrelated with the noise µ § i.e.

µ §	is nonsingular	(4.19)

µ §	(4.20)

Then we may try to estimate the parameters by exploiting these properties. Correlating both sides of the equation error model Eq. (4.4) with µ § using the data for µ § we obtain

µ §	(4.21)

Since µ § is assumed to be uncorrelated with the process noise µ §, for large values of µ § we have

µ §	(4.22)

Then the system of linear equations obtained from Eq. (4.21) gives rise to an estimate

µ §	(4.23)

or in the equivalent matrix equation form

µ §	(4.24)

where µ § and µ § are the same as in Eq. (4.3) while

µ §

This is the so called instrumental variable (IV) estimate of µ §. The elements of vector µ § are usually called the instrument variables. The scheme of the instrumental variable method is shown on Figure 4.5. Due to the definition of µ §, the estimation errors tend to zero when µ § tends to infinity, therefore the instrumental variable method gives consistent estimates. Evidently the instrumental variable estimate is a generalization of the least squares estimate since substituting µ § with µ § in Eq. (4.24) it reduces to Eq. (4.11).

There are many ways to choose the instrument variable vector. If Eq. (4.19) and (4.20) are satisfied all the choices will give consistent estimates; the differences lie in the corresponding covariances. Assuming that the process is described by Eq. (4.1), one possibility for choosing the instruments is the following form

µ §	(4.25)

where the signal is obtained by filtering the input

µ §	(4.26)

The coefficients of the polynomials µ § and µ § can be chosen in many ways. One special choice is let µ § and µ § be the a priori estimates of µ § and µ § respectively or the estimates obtained by applying first a simple least squares estimation. Another special case where µ § and µ §, then µ § becomes, after reordering of the elements

µ §	(4.27)

Note that reordering of the instruments µ § has no influence on the corresponding instrumental variable estimate.

µ §

Figure 4.5	The instrumental variable method

Example 4.2

Let us consider the system and the coloured noise experiments as in Example 4.1. The parameters of the same model is to be estimated now applying the instrumental variable method. Two cases are studied:

(a)	The instrument is constructed using the least squares estimates obtained in Example 4.1 and then µ §

where µ §.

(b)	The instrument is based on delayed inputs, i.e. µ § and then µ § which can be reordered of course.

The instrumental variable estimates obtained from 1000 samples are summarized in Table 4.2, the residuals for both cases are shown in Figure 4.6. The parameter estimates are significantly closer to the real values than those obtained with the least squares method. The residuals are much smaller compared to those given by the least squares estimate in an experiment under the same conditions. The application of the two different instruments practically provides the same estimates and therefore the same residuals.

Table 4.2	Estimated parameters in case of white and in case of coloured noise

| Parameter | Real value | Estimated value |  |
| --- | --- | --- | --- |
|  |  | (a) | (b) |
| µ § | -1.5 | -1.4989 | -1.4973 |
| µ § | 0.7 | 0.6997 | 0.6944 |
| µ § | 1.0 | 0.9827 | 0.9829 |
| µ § | 0.5 | 0.5155 | 0.5176 |

Figure 4.6	Residuals obtained with instrumental variable estimates using two instruments.

(a) Instrument is obtained from the least squares estimate.

(b) Instrument is the delayed input.

## 4.5 Maximum likelihood identification (ML) (Johansson, 1993; Åström, 1980; Zhu, 1993)

The maximum likelihood (ML) method is one of the few generally applicable methods for parameter estimation. The basic idea is to construct a function of the data and the unknown parameters called likelihood function. The estimate is then obtained as the parameter value which maximizes the function. The likelihood function is essentially the probability density function of the observations. Thus maximum likelihood identification means that we select the estimate which renders the given observations most probable.

Suppose that the observations are represented by a random vector variable µ § and denote the probability density function of µ § as

µ §	(4.28)

Here the unknown parameter vector µ § that describes the properties of the observed variable will be estimated using the observations of µ §. Then the likelihood function for the observed values can be expressed as

µ §	(4.29)

The likelihood function is a deterministic function of the parameter vector µ § once the numerical value of µ § is inserted; in contrast the probability density function is function of random variable µ § for fixed parameter vector µ §.

The estimate can be given then as

µ §	(4.30)

The principle of maximum likelihood method is simple and it can be shown that it gives asymptotically (when µ §) unbiased and efficient (minimum variance) estimates (for details see Söderström and Stoica, 1989).

Let us consider the system described by Eq. (4.3) and assume that µ § is normally distributed and it has µ § elements, furthermore its mean µ § and its covariance µ § are known. It is equally valid to use the probability density function of the noise since there is a one-to-one transformation between µ § and µ §. Then the corresponding multivariable Gaussian probability density function of µ § takes the following form

µ §	(4.31)

Under the assumption of Eq. (4.3) we get the likelihood function

µ §	(4.32)

We may consider Eq. (4.32) as a function of the parameter vector µ § and the corresponding equation error defined by Eq. (4.4). It is practical to take the logarithm of the likelihood function

µ §	(4.33)

Then at the maximum of the log-likelihood function we obtain the estimate µ §. In the case when the noise is a normally distributed white noise with a covariance matrix µ §, it is easy to show that the maximization of the likelihood function (Eq. 4.33) is equivalent to the minimization of the loss function

µ §	(4.34)

If µ § is unknown, maximization of Eq. (4.33) with respect to the parameters and µ § can be done separately. First minimize Eq. (4.34) with respect to µ §. The maximum of Eq. (4.33) with respect to µ § is then can be given in the following form

µ §	(4.35)

The problem of maximizing Eq. (4.33) can be solved using suitable numerical tools, like the Newton-Raphson method.

Let us take a more complex problem now, namely a system with coloured noise which is described in the form of an autoregressive moving average model with exogenous input (ARMAX)

µ §	(4.36)

where the noise covariance matrix µ § is assumed to be unknown. Then in case of normally distributed noise the log-likelihood function can be given as

µ §	(4.37)

where µ §, µ § and µ § µ §. The problem is that µ § are not known. It is therefore only feasibly to make approximate solutions by finding successively better estimates of the covariance matrix µ § and the parameters µ § through an iterative procedure. Further details of the method are discussed in Johansson (1993) and Åström (1980).

## 4.6 Prediction error methods (Söderström, 1989; Johansson, 1993)

The prediction error methods (PEM) predict µ § based on the previous data and the identified model, and then minimize the error between the predicted and real output. These methods are natural to use in context of time-series models of prediction and filtering. In general terms many methods of parameter estimation can be considered as minimization of errors of some predictions from measured data. However prediction error methods are more restricted and comprise a well defined class of identification methods.

Considering the our basic model given by Eq. (4.1) and neglecting the noise term the output µ § at time µ § can be predicted as

µ §	(4.38)

hence the equation error defined by Eq. (4.4)

µ §	(4.39)

can be interpreted as a prediction error where the one-step-ahead prediction µ § is based on the data up to and including time µ § and the model parameter vector µ §.

To formalize the method, the following general model structure is introduced

µ §	(4.40)

Assume that µ §, i.e. that the model has at least one pure delay from input to output. As a general form of the linear predictor, consider

µ §	(4.41)

which is a function of past data only if the predictor filters µ § and µ § constrained by

µ §	(4.42)

The predictor can be constructed in various ways for a given model (Eq. 4.40). Once the model and the predictor are given, the prediction errors are computed according to Eq. (4.39). The parameter estimate µ § is then chosen to make the prediction errors µ § small. The basic principle of the prediction error method is illustrated on Figure 4.7.

µ §

Figure 4.6	Scheme of the prediction error method

To define the prediction error method the user has to make the following choices:

(1)	Choice of model structure. This concerns the parametrization of µ § and µ § in Eq. (4.38)as a function of µ §. For algorithmic reasons the transfer functions are generally factorized into numerator and denominator polynomials. In the context of identification, there are a few often used transfer function models:

(a) The output error model

µ §	(4.43)

contains no assumption on the spectrum of the noise. In such a case the prediction error method is often called output error method (OEM).

(b) The Box-Jenkins model

µ §	(4.44)

contains a noise model with a white noise sequence µ § filtered through the transfer function µ §. An attractive property of the Box-Jenkins model is that it yields separate descriptions of the input-output relationship between µ § and µ § and the noise spectrum model.

(c) The generalized model structure can be used to treat transfer function models as well as difference equation models:

µ §	(4.45)

This structure is too general in most of the cases, therefore one or several of the five polynomials would be fixed to unity in applications. this way we can obtain e.g. the output error or the Box-Jenkins model also.

(2)	Choice of predictor. This concerns the filters µ § and µ § in Eq. (4.41), once the model is specified. The filters can in principle be chosen in many ways. The most common way to let the filters be an optimal mean square predictor. This means that the filters are chosen so that under the given model assumptions the prediction errors have as small variance as possible.

Let us consider the following first-order (ARMAX) model

µ §	(4.46)

where µ § is a zero mean white noise. It can be shown (see Söderström, 1989) that a convenient expression of the optimal predictor is

µ §	(4.47)

The prediction error can be expressed in a similar recursion

µ §	(4.48)

The initial value µ § can be selected as zero, which is a sufficient solution for sufficiently large µ §.

(3)	Choice of criterion. This concerns a scalar-valued function of all the prediction errors µ §, which will assess the performance of the predictor used. This criterion is to be minimized with respect to µ § in order to choose the best prediction in the class considered. In case of one output the criterion (loss function) can be defined as the sum of squares of the prediction error, while in case of multi-output systems as scalar mapping of the covariance matrix

µ §	(4.49)

where µ § is a scalar-valued function and it is usually selected as the trace of the weighted covariance matrix or the determinant of it.

Analysis of the prediction error method shows that the loss function converges to a minimum point and the estimate is consistent if the following conditions are satisfied:

·	The data µ § are stationary processes.

·	The input are persistently exciting.

·	The Hessian µ § is nonsingular at least around the minimum point of µ §

·	The filters µ § and µ § are differentiable functions of the parameter vector µ §.

In most cases the minimum of µ § cannot be found analytically. For such cases the minimization must be performed using a numerical search routine. A commonly used method is the Newton-Raphson algorithm:

µ §	(4.50)

Here µ § denotes the µ §-th iteration point in the search. The sequence of scalars µ § is used to control the step length. Strictly for the Newton-Raphson algorithm µ §, however it is often necessary to use different, in practice a variable step length is often necessary to improve the convergence of the method.

Example 4.3

Let us consider the data obtained in Example 4.1 (b) with colored noise for 1000 sampling periods. The parameters of the model was estimated using the prediction error method in two ways: (a) a simple output error model was used i.e. µ § in the generalized model structure (Eq. 4.45), (b) a difference equation model with the same structure as the system were used i.e. µ § in the generalized model structure. The results are summarized in Table 4.3. Ignoring the noise model, the estimated parameters are practically identical to those of the original system in case (a) and very close to them in case (b). The residuals are not plotted here since they are almost the same as in Figure 4.6.

Table 4.3	Parameters obtained with the prediction error method

| Parameter | Real value | Estimated value |  |
| --- | --- | --- | --- |
|  |  | (a) | (b) |
| µ § | -1.5 | -1.4981 | -1.4988 |
| µ § | 0.7 | 0.6987 | 0.6991 |
| µ § | 1.0 | 0.9903 | 1.0008 |
| µ § | 0.5 | 0.5149 | 0.5038 |
| µ § | -1.0 | - | -1.7363 |
| µ § | 0.2 | - | 0.8371 |

## 4.7 Recursive methods of parameter estimation (Söderström, 1989; Johansson, 1993; Isermann, 1991; Ljung, 1987)

In recursive identification methods, the parameter estimates are computed recursively in time. This means that the estimate µ § is computed as a "modification" of the estimate µ § using the new measurements µ § and µ §. The recursive methods of parameter estimation basically have been developed for real-time identification purposes. Their main applications are the different adaptive systems and also fault diagnosis. These applications requires that the action or evaluation use the most recent model of the system.

Recursive identification methods have the following general features:

·	They can be easily modified for time-varying systems. Thus they allows that the model follows the changes of system parameters or the changes due to changing operating conditions in case of nonlinear systems.

·	Their primary memory requirement is quite modest and not increasing with time, since not all data are stored.

·	They are principally derived as an approximations of nonrecursive methods discussed in the previous sections.

### 4.7.1 Recursive least squares method

To derive the recursive least squares method let us consider the nonrecursive least square estimate given by Eq. (4.11) and rewrite it in form of sums

µ §	(4.51)

The argument µ § is used to emphasize the dependence of µ § on time. This expression then can be computed in a recursive fashion. Introduce the notation

µ §	(4.52)

Since trivially

µ §	(4.53)

Then µ § can be expressed as

µ §	(4.54)

Then the recursive least squares estimate takes the following form

µ §	(4.55)

where the last factor should be interpreted as a prediction error, while the correcting factor can be considered as a weighting or gain factor determining how much the prediction error will modify the different elements of the parameter vector.

Computing µ § according to Eq. (4.53) requires a matrix inversion at each step. Using the matrix inversion lemma µ § and taking µ §, µ §, and µ § a more efficient expression, avoiding matrix inversion, can be obtained

µ §	(4.56)

where

µ §	(4.57)

which is nothing else but the gain in Eq. (4.55) and then the expression for the estimation takes the following form

µ §	(4.58)

Initial values

The recursive algorithm requires appropriate initial values for the parameter vector and matrix µ §. Since µ § is proportional to the covariance matrix of µ § it is reasonable to take for µ § an a priori estimate and to let µ § reflect the confidence in this initial estimate. If µ § is small then µ § will be small for all µ § and the parameter estimates will therefore not change too much from µ §. On the other hand, if µ § is large, the parameter estimates will quickly jump away from µ §. Without any a priori information it is a common practice to take

µ §	(4.59)

where µ § is a "large" number.

Another solution, if the application allows, is that to apply first the nonrecursive least squares algorithm on a small number of data and use the estimate as µ § and calculate µ § from the same data. In this case the subsequent recursive estimates are exactly the same as if we apply the nonrecursive method on all the data measured up to time µ §.

Forgetting factor

Many applications require tracking of the time-varying parameters of the system. One method allowing changes of the estimated parameters is to introduce some kind of weighting in the loss function. This way one can control how much the estimation is based on old or more recent measurements i.e. how fast the old measurements are "forgotten". The weighting or forgetting should be set in accordance to the pace of parameter changes.

One commonly used method is to use a forgetting factor µ § to provide an exponentially decreasing weight. The loss function to be minimized takes the following form

µ §	(4.60)

where µ § should be chosen somewhat less than µ § (it is recommended to choose the value for µ § between µ § and µ §).

Then the estimation algorithm can be rederived applying Eq. (4.60) and the following expression of the recursive least squares method with forgetting factor is obtained

µ §	(4.61)

µ §	(4.62)

µ §	(4.63)

For µ § the common recursive least squares estimate is obtained.

The forgetting factor can also be used to improve the transient behavior of the recursive least squares algorithm. A smaller µ § allows the parameter estimates to change quickly, then having µ § to tend exponentially to µ §. This can be implemented as a recursion for µ § e.g. in the formµ §, some practical choice for the parameters is µ § and µ §.

Example 4.4

The parameters of the system with colored noise in Example 4.1 (b) were determined using the recursive least squares method with forgetting factor. The same data and the same model was used as in Example 4.1. To illustrate the effect of the forgetting factor two cases were studied: (a) µ § and (b) µ §. The initial values µ § and µ §

The sequence of the estimated parameter values are shown on Figure 4.7.

Figure 4.7	Recursive least squares estimation with forgetting factor

(a) µ §

(b) µ §

After a short initial period the estimated parameters tend to those of the original system. In case (a) the estimated parameters stabilizes close to the true parameter values while in case (b) the estimates keeps changing in some neighborhood of the true values due to the low value of the forgetting factor.

### 4.7.2 Unified recursive parameter estimation algorithm

To utilize the advantages of the recursive calculation and to allow real-time processing the recursive version of most of the parameter estimation methods have been developed. These methods can be represented in a general, unified framework:

µ §	(4.64)

µ §	(4.65)

µ §	(4.66)

The methods differ only in the parameter vector, the data vector and in the correcting vector. Here, beside the recursive least squares method (RLS), the recursive instrumental variable method (RIV) and the recursive maximum likelihood methods (RML) are presented. The corresponding variables of Eq. (4.64-4.66) are summarized in Table 4.4. To obtain the expression with a forgetting factor the following modifications are required:

(1)	The µ § in the denominator of µ § is replaced by µ §. E.g. for the recursive instrumental variable method

µ §	(4.67)

(2)	µ § is divided by µ §. E.g. for the recursive least method

µ §	(4.68)

A comparison of recursive parameter estimation methods with respect to the performance of the estimates, the reliability of the convergence and the computational effort is given by Isermann (1991). The properties of the methods presented above are summarized in the following:

RLS:	Applicable for small noise/signal ratios, otherwise gives biased estimates. reliable convergence. Relatively small computational effort.

RIV:	Good performance of parameter estimates. To accelerate the initial convergence, starting with RLS is recommended. Larger computational effort than RLS.

RML:	High performance of parameter estimates, if the noise model fits. Slow convergence in the starting phase. Noise parameters are estimated, but show slow convergence. Larger computational effort than RLS and RIV.

Table 4.4	Unified recursive parameter estimation algorithms (based on Isermann, 1991)

| Method | µ § | µ § | µ § | µ § | µ § |
| --- | --- | --- | --- | --- | --- |
| RLS | µ § | µ § | µ § | µ § | µ § |
| RIV | as RLS | as RLS | µ § | µ § | µ § |
| RML | µ § | µINCRUSTAR Equation µ §§ | µINCRUSTAR Equation µ §§ | µINCRUSTAR Equation µ §§ | µ § |

# 5. Model structure selection and model validation

(compiled from: Ljung, 1987; Söderström, 1993; Johansson, 1993; Isermann, 1980; Gustavsson, 1972)

The choice of an appropriate model structure is very crucial for a successful identification application. This choice must be based both on an understanding of the identification procedure and on insights and knowledge about the system to be identified. Once the model structure has been chosen, the identification procedure provides a particular model in this structure. This model may be the best available one, but the crucial question is whether it is good enough for the intended purpose. Testing if a model is appropriate is the step of model validation. Naturally such techniques are closely related to the choice of the model structure.

## 5.1 General aspects of model structure selection (Ljung, 1987; Isermann, 1980; Gustavsson, 1972)

The choice of model structure certainly has a considerable effect on both the quality of the resulting model and the computational price for it. The quality of the model can, e.g. be measured by a mean-square error criterion. Then the best model structure is a trade-off between flexibility and parsimony. Flexibility means that the model structure has good capabilities of describing different possible systems and one way to obtain it using many parameters. Parsimony, on the other hand, requires not to use too many parameters.

To start the solution of an identification problem, three important aspects must be considered:

·	type of model set

·	size of the model set or model order selection

·	identification method

Selection of the type of model set involves the choice between different classes of models, like between nonlinear and linear models or between input-output, black-box and physically parametrized state-space models, and so on. The choice of which type of model to use is quite subjective and involves several issues that are independent of the data set. The choice is usually a trade-off between different factors, like model quality and modeling efforts. To obtain a good fit with few parameters, the a priori knowledge and also some intuition should be utilized. A priori information can typically best be incorporated into continuous-time models. This also means that the evaluation of µ § and therefore the minimization of the loss function requires more programming effort and computing time. Aspects of algorithmic complexity as well as the shape of the criterion function thus favor black-box models. Sophisticated model structures should be used only if simpler ones do not pass the model validation. Especially linear regression models lead to simple and robust minimization schemes. Nonlinear transformation of data - e.g. in case of actuators and sensors - can also be considered to fit a linear model.

Model order selection or determination of the size of the model set involves selecting the structural parameters of the model class. This step usually requires some evaluation of the data set, however the intended model application and some physical insight often suggest the range of model orders to be considered. The size of data set and the data quality also indicates how many parameters it is feasible to estimate. Another related problem is the number of applicable time scales. If the system contains widely separated time constants of interest then more models should be built, each covering a proper part of the frequency range with a suitable sampling interval.

Selection of identification method is strongly depends on the selected class of models. As it was shown in the earlier chapters, a linear system can be represented in many different ways, e.g. by their transient or frequency responses or by parametric models. It is relatively simple to transform one representation to another. However, these transformations may be very badly conditioned in the sense that small errors in one representation can give rise to large errors in another representation. The conclusion is again that the choice of the method is intimately related to the purpose of the identification. With respect the parameter estimation methods and given model order the following items are important for the selection of the method:

·	the performance with regard to the accuracy of the identified model (model errors) dependent on time (convergence properties).

·	the computational effort (computer storage, computation time)

## 5.2 Preliminary data analysis for model structure selection (Ljung, 1987)

Certain testing and evaluation performed on the available data give insight into possible and suitable model structures. These techniques do not involve the determination of a complete model of the system. For parametric methods the problem is reduced to the selection of the model order. Some methods that are based on preliminary data analysis are discussed in the following.

(1)	Examining the spectral analysis estimate of the transfer function. Spectral analysis provides information about the interesting frequency range of process and the noise level which help the design of input signals and the corresponding filters. A nonparametric estimate of the transfer function µ § gives reasonable information about the resonance peaks and the phase shift. All this gives a hint to what model orders are required for an adequate description of the (interesting part of the) process dynamics.

(2)	Testing ranks in covariance matrices is a good method for estimating  the order of the system. Suppose that the true system is described in the following form

µ §	(5.1)

and µ § is the "true" order of the system. Let us take a model of order µ § and then the corresponding data vector is

µ §	(5.2)

Consider first when µ §. Then Eq. (5.1) implies that the covariance matrix

µ §	(5.3)

will be nonsingular for µ § (provided that µ § is persistently exciting) and singular for µ §. µ § could be used as a test quantity for the model order.

In case a noise is present in Eq. (5.1) the covariance matrix can still be used with a suitable threshold, provided that the signal-to-noise ratio is high. Otherwise the use of an "enhanced" matrix is suggested

µ §	(5.3)

where µ § is the estimated influence of the process noise on the covariance matrix.

A better alternative, when the influence of µ § is not negligible, is to use other correlation vectors. If µ § and µ § are uncorrelated, we could use

µ §	(5.4)

and then the covariance matrix

µ §	(5.5)

is nonsingular for µ § and singular for µ §.The expectation should be replaced by the sample mean and then a usable test quantity is obtained.

(3)	Correlating the variables another method for determining the model order since it provides information whether to include one more variable in the model structure or not. This variable could be µ § or a measured possible disturbance variable. In any case the question is whether this new variable has anything to contribute when explaining the output variable µ §. This can be measured by the correlation between the new variable and the output variable. However, to discount the possible relationship between them, already accounted for by the smaller model structure, the correlation should be measured between the new variable and what remains to be explained, i.e. the residuals µ §. This is known as partial correlation in regression analysis.

## 5.3 Criteria for comparing model structures (Isermann, 1980; Söderström, 1993; Ljung, 1987; Johansson, 1993)

A natural approach to search for a suitable model structure is simply to test a number of different ones and compare the resulting models. A few commonly used order testing methods are

·	loss function tests

·	pole-zero cancellation tests

·	residual tests

Loss function tests

The most simple method is just regard the loss function µ § in dependence of the order of the model. For increasing model order the loss function first decreases and then remains constants or changes slightly. Other methods are based on the statistical test of the loss function or on the evaluation of different criteria assessing the model structures under study.

(1)	A statistical method to test if the loss function reduces significantly with increasing model order is the F-test. This test is based on the statistical independence of µ § and µ § where the superscripts denotes models with number of parameters µ § and µ § respectively. To test if the reduction of the loss function is significant when the number of parameters is increased from µ § to µ § the following test quantity is used

µ §	(5.6)

This quantity is exactly µ § distributed. Then for sufficiently large number of samples µ §, the following holds: if µ § then the hypothesis that significant reduction of the loss function was achieved by increasing the number of parameters is accepted at µ § confidence level. The significance level µ § in practice could range from µ § to µ §.

(2)	Criteria with complexity terms may be obtained by supplementing the loss function with some extra term penalizing for model complexity. Then the model structure giving the smallest value of this criterion is selected.

Akaike's information criterion (AIC) decreases as the loss function decreases and it grows as the number of parameters µ § increases. If the loss function is defined as

µ §	(5.7)

the criterion takes the following form

µ §	(5.8)

As the expected residual variance decreases with increasing µ § for nonadequate model complexities, there should be a minimum around the correct number of parameters. It is observed that AIC may overestimate the model order.

Another approach is to analyze the prediction performance when the estimated model is applied to another data set. It can be suspected that a large overparametrized model might poorly predict the behavior of the new data set. The idea is to separate the contributions to the prediction error into noise variance and model error terms. The final prediction error criterion (FPE), suggested by Akaike applies the idea in the following from

µ §	(5.9)

It is sometimes observed that the FPE tends to underestimate the correct order of the system.

An attractive property of decision criteria AIC and FPE is that the minimum of the test statistics indicates what model to choose therefore these statistics need not be compared against some statistical significance level.

Pole-zero cancellation tests

If the assumed order of model µ § is higher than the order of the real process µ § additional pole-zero pairs result which at least approximately cancel each other. This effect can be used for order testing by calculating the roots of the polynomials µ § and µ § for different model orders µ §.

Residual tests

Considering linear prediction error models of a system, the residuals obtained as

µ §	(5.10)

represent a kind of disturbance that would explain the mismatch between the observed data and the behavior of the estimated model. In general the residuals will be white or near white if the model is in good agreement with the true system. A sequence of residuals which still exhibit some structure would then indicate that either the modeling or the identification is not complete. If the model is correct and if the method prerequisites are satisfied, then the residuals should be structureless; in particular, they should be uncorrelated to any other variable including the inputs and the outputs. This is the assumption on which the residual tests are based.

A simple check is to plot the residual versus the fitted values; such a plot should not reveal any obvious pattern. Another valuable diagram is the histogram of the residual amplitudes, which reveals distributions that differ from normal distribution.

Based on the statistical analysis of the residuals µ §, several statistical tests like the autocorrelation test, the cross-correlation test, test of normality and so on can be applied. For details see Johansson (1993).

## 5.4 Model validation (Isermann, 1980; Ljung, 1987)

The parameter estimation procedure picks out the "best" model within the chosen model structure. The crucial question is whether the estimated model is "good enough" for the intended purpose. Model validation means to check if the identified model agrees with the real process behavior, taking into account the limitations of any identification method and the final goal of the model application. The problem therefore has several aspects:

1. Does the model agree sufficiently well with the observed data?

2. Is the model good enough for the required purpose?

3. Does the model describe the "true" system?

The method to answer these questions is to confront the model with as much information - like a priori knowledge, experimental data and experience - about the true system as it is practical.

Since in an identification application the most natural entities with which to confront the model is the data themselves, model validation techniques tend to focus on question 1. While question 3, ignoring test problems, is philosophically impossible to answer, question 2 is an important practical issue. This ultimate validation is the most important test, i.e. to evaluate whether the problem that motivated the modeling task can be solved using the obtained model. If a controller based on the model satisfies the control requirements, then the model was a "valid" one regardless of any formal aspects. However it is often impossible, costly or dangerous to test all possible models with their intended use. Instead, the confidence in the model is developed in other ways. The two main methods are: the verification of the a priori assumptions and the verification of the input-output behavior.

Verification of a priori assumptions

The test whether the a priori assumptions of the identification method are true can be performed as follows:

·	Linearity: Comparison of models obtained for different input amplitudes. Comparison of models with measured transient functions in both directions.

·	Time variance: Comparison of models for different data sets.

·	Residuals: Are the residuals statistically independent, with zero expectation? Are they independent from the input signal?. Checking the signals: are they outliers?

·	Bias, drift: Averaging of output. Comparison of models with and without drift elimination.

·	Input signal: Can be measured without noise? Is it persistently exciting?

·	Covariance matrix of the parameter estimates: Do the variances and covariances decrease with increasing number of samples?

Verification of the input-output behavior

A final judgment of the identified model is obtained by comparing the measured and the model predicted input-output behavior. This can be done by:

·	Comparison of the measured µ § and the calculated output signals µ §:

-	for the input µ § used for the identification,

-	for other input signals like steps or impulses.

·	Comparison of the crosscorrelation function based on the measured signals and on the model

Another way is cross-checking, that means the verification of the identified model for another set of measurements. The comparison between the observed data and the model output is usually revealing with regard to model anomalies not previously detected.

# References

Åström, K. J. - Eykhoff, P.: System identification - A survey, Automatica, 7, p. 123-162, 1971.

Åström, K. J.: Maximum likelihood and prediction error methods, Automatica, 16, p. 551-574, 1980.

*	Bohlin, T.: Interactive system identification: prospects and pitfalls, Springer Verlag, 1991.

*	Brdys, M. A. - Malinowski, K.: Computer aided control system design, World Scientific, 1994.

Evans, D. C. - Rees, D. - Jones, D. L.: Design of test signals for identification of linear systems with nonlinear distortion, IEEE Trans. on Instrumentation and Measurement, 41, 6, p. 768-774, 1992.

Fasol, K. H. - Jorgl, H. P.: Principles of model building and identification, Automatica, 16, p. 501-518, 1980.

Godfrey. K. R.: Correlation methods, Automatica, 16, p. 527-534, 1980.

Godfrey, K. - Jones, P.: Signal processing for control, Springer-Verlag, 1986.

*	Gustavsson, I: Comparison of different methods for identification of industrial processes, Automatica, 8, p. 127-142, 1972

Isermann, R.: Practical aspects of process identification, Automatica, 16, p. 575-587, 1980.

Isermann, R.: Digital control systems, Vol. II, Springer-Verlag, 1991.

Johansson, R.: System modeling and identification, Prentice Hall, 1993.

Landau, I. D.: System identification and control design, Prentice Hall, 1990.

Ljung, L.: System identification - Theory for the user, Prentice Hall, 1987.

*	Ljung, L. - Soderstrom, T. - Gustavsson, I.: Counterexamples to general convergence of a commonly used recursive identification method, IEEE Trans. on Automatic Control, AC-20, 5, p. 643-652.

Schoukens, J. - Pintelon, R.: Identification of linear systems, Pergamon Press, 1991.

Strejc, V.: Least squares parameter estimation, Automatica, 16, p. 535-550, 1980.

Söderström, T. - Stoica, P.: System identification, Prentice Hall, 1989.

Rake, H.: Step response and frequency response methods, Automatica, 16, p. 519-526, 1980.

*	Ramirez, W. F.: Process control and identification, Academic Press, 1994.

Zhu, Y. - Backx, T.: Identification of multivariable industrial processes - for simulation, diagnosis and control, Springer-Verlag, 1993.

Note:	Papers and books marked with an asterisk are not cited in the text, however they are closely related to the topic and they are recommended as further readings.