2.3. Electrical Systems

The electrical theory can be categorised as direct-current (dc) or alternating-current theory. On the other hand it can be lumped or distributed-parameter model. A lumped model is used when the size of the electrical element is small compared to the signal’s wavelength. For example a relatively short wire can be considered lumped at a point and represented by the resistant element. In this part the relations of resistance, capacitance and inductance will be discussed.

Table 2.3.1 . Circuit elements, quantities and units

| Quantities. | Symbols | Units | Representation |
| --- | --- | --- | --- |
| Resistance | R | W (Ohm=volt/ampere) |  |
| Capacitance | C | F (farad=coulomb/volt)) |  |
| Inductance | L | H (henry=volt-sec/amp) |  |
| Voltage | v | v (volt) |  |
| Charge | Q | coulomb ( N-m/volt) |  |
| Current | i | ampere (coulomb/sec) |  |
| Flux | F | volt-sec |  |
| Battery (plus int.resistance) | -------- | --------- |  |
| Ground | -------- | --------- |  |

Some values for typical used circuits elements:

Resistance values range from 1000 W, 100000 W; capacitance values range from 10−12Fto10−4F. Most of inductors have an inductance less then 1 H. The relation between the stored charge

and the voltage produced is described by the following equation :

| v(t)=1C0tidt+Q0C | (2.3.1.) |
| --- | --- |

The relation between voltage and current for an inductor is:

| i(t)=1L0tvdt | (2.3.2.) |
| --- | --- |

2.3.1. Circuit Laws.

For electrical circuits Kirchhoff’s current and voltage laws are used. The current law or node law states that the sum of the currents entering the node equals the sum of the currents leaving the node. The positive direction of current flow is opposite to the flow of negative charge. The voltage law (loop law) states that the instantaneous algebraic sum of the voltages around any loop in a circuit is zero which really means that the voltage impressed on a closed loop equals the sum of the instantaneous voltage drops in the rest of the loop. The point in the circuit at which the voltage is zero is indicated by the ground symbol given in table 2.3.1.

The voltage drop across a resistance is :

| vR=R⋅i | (2.3.3.) |
| --- | --- |

The voltage drop across a capacitor is :

| vC=QC=1C0tidt+Q(0)C | (2.3.4.) |
| --- | --- |

The voltage drop across an inductor is:

| vL=L⋅didt | (2.3.5.) |
| --- | --- |

Example 2.3.1. Figure 2.3.1.

Develop a model for the voltage e across the capacitor in a circuit consisting of a voltage source v , a capacitor (capacitance C ) and a resistor (resistance R ), called the series RC circuit.

Solution : First step: Setting up the Model: Kirchhoff’s voltage law states that the voltage impressed on a closed loop equals the sum of the instantaneous voltage drops in the rest of the loop, that means:v=vc+vr relation which can be written as v=Ri+e. The voltage e across the capacitor C can be expressed with the current i in the following way:e=1Cotidt+Q0C.

Second step. General solution. Differentiating this with respect to time yields: dedt=iC, and from the first relation we have : i=v−eR.From these last two relations we obtain: dedt=v−eRC, which can be written as follows: Cdedt=-1Re+1Rv.This is the general solution.

Figure 2.3.1. RC-circuit

i

-

+

e

C

v

R

Exercise 2.3.2.

Develop models for the circuits shown in Figure 2.3.2. The input is the voltage v the output is the current i .

Figure 2.3.2. Electric circuits.

i

d.)

c.)

L

C

R

v

L

C

i

v

b.)

C

R

v

a.)

L

v

Solution of 2.3.2.d

First Step. Setting up the Model.

Kirchhoff’s voltage law states that the voltage impressed on a closed loop equals the sum of the instantaneous voltage drops in the rest of the loop. That means: v=vR+vC+vL , which yields: v=Ri+QC+Ldidt , wherei=i(t)and Q=Q(t)and Q(t)=1C0tidt+Q0C.This yields :v=Ri+1C0tidt+Q0C+Ldidt. Differentiating both sides with respect to time we obtain:.

| Ld2idt2+Rdidt+1Ci=dvdt | (2.3.6.) |
| --- | --- |

This is a second-order differential equation whose general solution depends on two constants. Assuming that v (0) and Q(0) are given we can determine both constants, and can obtain the particular solution.

Example 2.3.3.

Develop a model for the following RLC circuit shown in figure 2.3.3. The inputs are R,L,C , v, the output is the current i flowing in the circuit. Determine the particular solution for the problem, assuming that no current is flowing and no charge is at t=0 .

Figure 2.3.3. RLC-circuit.

L

C

R

v

R = 1010 (W)

L = 1 (H)

C = 0.0001 (F)

v = 99 sin5t (V)

Solution. First Step. Setting up the Model.

Kirchhoff’s voltage law states that the voltage impressed on a closed loop equals the sum of the instantaneous voltage drops in the rest of the loop. That means: v=vR+vC+vL. According to Exercise 2.3.3. we obtain (2.3.6.) a second-order differential equation.

From the initial conditions (no current and no charge on the capacitor at t=0) we obtain: i(0)=A=0 and i’(0)=B=0 , where i’ means the derivative of i with respect to time.

Using the Laplace Transformation we obtain the following Laplace Transforms:

L[i(t)]=0¥i(t)⋅e−stdt:=I(s), L[i'(t)]=sI(s)−i(0)=s⋅I

L[i''(t)]=s2I(s)−si(0)−i'(0)=s2I. L(cosat)=ss2+a2,L(sinat)=as2+a2

where s is a positive real parameter. Substituting the quantities for their given values in (2.3.6.) we obtain the following  differential equation:

i''+1010i'+10000i=495cos5t. Using the previous Laplace Transforms of the functions in this equation we obtain the following algebraic equation with unknown I with respect to s: s2I+1010sI+10000I=495s2+25. Solving it with respect to I and writing the obtained fraction in terms of partial fractions we have: I(s)=−0.04s+10+0.0005s+1000+0.04ss2+25+0.1s2+25.

Taking the inverse Laplace transforms of both sides we have:

i(t)=-0.04e−10t+0.0005e−1000t+0.04cos5t+0.02sin5t. The first two terms die down in a very short time ( this is the transient or unsteady state current) and what remains (the last two terms )is the steady state current, which is a periodic function  and can be written as:

0.044sin(5t+1.1).

Example 2.3.4.

Develop models for the circuits shown in Figure 2.3.5. The input is the voltage v  the output is the voltage e.

e

Figure 2.3.5.

c.)

b.)

a.)

R2

C

R1

v

C

e

R2

R1

v

R

L

e

v

Solution a.).First step. Setting up the Model.

According to the voltage law the applied voltage equals the voltage drops across the elements in the circuit. That means: v=vR+vL which yields : v=Ri+Ldidt  The output that is looked for is e=Ldidt, and the only input is v .Then we have e=v−Ri. We have to eliminate i, and to achieve this goal we differentiate this relation and obtain: dedt=dvdt−Rdidt. From this relation  multiplying it by L and dividing it by R we obtain Ldidt=LRdvdt−LRdedt. Now substituting Ldidt for e, we have dedt+RLe=dvdt. This is the mathematical model corresponding to the problem.

Solution c.).First step. Setting up the Model.

According to the voltage law the applied voltage equals the voltage drops across the elements in the circuit.  That means: v=R1i+QC+R2i. The output that is looked for is e=R2i+QC. We know that Q(t)=1C0tidt+Q(0)C. This yields : e=1C0tidt+Q(0)+CR2i. Differentiating it with respect to time we have dedt=1Ci+R2didt=v−eCR1+R2didt. In order to eliminate the time derivative of the current from this relation we consider the following relation:R1i=v−e, which differentiating with respect to time yields: R1didt=dvdt−dedt. This results: dedt=v−eCR1+R2R1dvdt−dedt. From this relation we obtain: 1+R2R1dedt=v−eCR1+R2R1dvdt.

Example 2.3.6.

Develop a model for the circuit in figure 2.3.5. where the voltage source is a battery. The inputs are v=70V,R1=10Ω,R2=20Ω,R3=40Ω.The outputs are i1,i2,i3.

Figure 2.3.6.

v

Solution. First Step. Setting up the Model.

We will decompose the given circuit in two loops as follows, and will apply the Kirchoff’s voltage Law (or loop law) for the obtained loops:

v

loop 2

loop 1

v

Now we can write: in loop 1: v=v1+v2=R1i1+R2i2, in loop 2 : v=v1+v3=R1i1+R2i2 . According to Kirchhoff’s current law (or node law ) we also have :i1=i2+i3.

Second Step: We determine the particular solution. We can set up the following simultaneous equations where the quantities have been substituted for their values:

70=(10+20)i2+10i370=10i2+(10+40)i3

From this equations we obtain :i1=3(A),i2=2(A),i3=1(A).

Third Step. Checking the Results.

Exercise 2.3.7.

Develop a model for the following electrical networks, shown in Figure 2.3.7.a. and 2.3.7.b, and find the currents, assuming that all charges and currents are zero when the switch is closed at t=0.

Figure 2.3.7.b.

i2

i2

i1

i1

R2

L2

L1

R1

v

Figure 2.3.7.a.

i2

i2

i1

i1

C=0.25(F)

R2=6 ohms

R1=4ohms

L=1(h)

v=12(V)

Solution a. Setting up the Model.

We have denoted by i1 the current in the left-hand loop and i2 the current in the right-hand loop. The mathematical model is obtained from Kirchhoff’s voltage law used for the left-hand, respectively for the right-hand loop. The left hand loop yields :di1dt+4(i1−i2)=12

The right hand loop yields :6i2+4(i1−i2)+4i2dt=0. Differentiating with respect to time we obtain:−4di1dt+10di2dt+4i2=0. After some algebraic operations we obtain the following system of equations:di1dt=-4i1+4i2+12;di2dt=-1.6i1+1.2i2+4.8. This system can written in matrix form: dIdt=A⋅I+a , where I=(i1,i2)T,a=(12;4.8)T. We substitute I for :I=xeλt+y, where x=(x1,x2)T and  y=(y1,y2)T. we obtain: xλeλt=Axeλt+Ay+a. The two unknown x and y can be determined from the following two conditions: Ax=λx and Ay+a=0. From the second one we obtain :y=-A−1a=-(−11.61.2−41.6−4⋅124.8=30. The equation Ax=λx will the following values : λ1=-2;λ2=-0.8 (called eigenvalues) and corresponding to these scalars we obtain the vectors: x1=21;x2=10.8 (called eigenvectors).

Than the general solution of the system is :I=C1x1e−λ1t+C2x2e−λ2t+a(matrix form) , and the scalar form: i1=2C1e−2t+C2e−0.87+3;i2=C1e−2t+0.8C2e−0.8t. The constants can be determined from the initial conditions, namely no current and no charge in the system when the switch closed. This results: i1=-8e−2t+5e−0.8t+3;i2=-4e−2t+4e−0.8t. That means that after a short period the transient will be zero and the steady-state current will be :i1=3;i2=0 which means that only in the first loop flows current , after the capacitor has been charged.

Solution b. Analogously we have :di1dt=-R1(1L1+1L2)i1−R2L2i2+(1L1+1L2)v, respectively: di2dt=-R1L2i1−R2L2i2+1L2v.