MB913

STM32-DISCOVERY

PCB Manufacturing Specification

# Modification History

NOTE: Change bars at the edge of each page show changes between revisions of document.

Please note that Section 1 Modification History on page 2 should only be used as a guide to the document changes and the whole document should be referenced for complete changes.

Revision  A

Created on 27/01/2010 by mr chen

# Contents

1	Modification History	2

2	Contents	3

3	PCB Manufacturing Specifications	4

3.1	Stack up description	4

3.2	PCB overview	4

4	PCB marking	5

5	PCB milling diagram	5

6	PCB Drill schedule	5

# PCB Manufacturing Specifications

## Stack up description

Stack up is a follow:

Artwork 1: Top-silkscreen

Artwork 2: Top-solder-mask

Artwork 3: Top-paste-mask

Artwork 4: Top-layer

Artwork 5: Bottom-layer

Artwork 6: Bottom-paste-mask

Artwork 7: Bottom-solder-mask

Artwork 8: Bottom-silkscreen

Artwork 9: Drill-drawing-layer1-2

Artwork 10: NC-drill

Describe here the pcb build up with all the layers (external copper, core, pre preg, inner copper) , their thickness and the material

## PCB overview

| Number of layers | 2 |
| --- | --- |
| Minimum track width | 152.4µm |
| Minimum spacing between tracks/pads | 152.4µm |
| Minimum hole size, mechanical and laser drill | mechanical drill: 406.4 µm; |
| Bare board test required (Single or Double sided) | Double sided |
| Silkscreen colour | White |
| Does the board have surface mount components | Top surfaces only |
| Plating type: Chemical Gold plating (See note), Selective Tin/lead, selective NIAu, Tin HAL, Chemical Tin | Immersion Gold |
| Legislation | PCB must comply RoHs legislation |
| Solder resist type process (photo imageable, LDI), colour | Photo imageable,Green |
| PCB material FR4 HTG xxx | FR4 HTG 175 |
| Copper weight (Outer) | 1oz |
| Copper weight (Inner) | 1oz |
| PCB Thickness and tolerence | 1.6mm +/- 0.15 mm |
| PCB size Panel size (refer to file pcb panel.gbr or .dxf) | 43.18mm *84.6 mm 215.90mm * 84.6mm |
| Information: Initial PCB manufacturer name | Chinafastprint |
| Controlled impedance lines (value and layers) | e.g. 45 ohm between layer 1-2 e.g. 50 ohm between layer 7-8 |
| controlled impedance line sample required |  |

Notes

1 The following standard PCB manufacturing edits are accepted: teardrop addition, non-functional pad removal, addition of copper in waste areas

2 If gold plating required use selected nickel gold plating, a flash of nickel at 5 micron and gold at 0.1micron.

Gold plating is also called Immersion Gold

3 Silkscreen clipping is acceptable as long as no pin 1 markers are removed.

4 Solder mask removals around vias drill holes are recommended. solder mask pads for the vias as follow: hole size+0.1mm

5 the initial PCB manufacturer indicates can eventually be used for manufacturing the series; this saves the cost of the tooling. This is to be balanced with the volume and unit cost capabilities.

6 List all the controlled impedance lines with the required impedance (single line or differential lines). E.g. USB lines between IC4 pin 3 and IC14 pin 15 & IC4 pin 4 and IC14 pin 16, differential 90 ohm

7 If “controlled impedance line sample required” answer is YES, the pcb manufacturer must manufacture a sample for each board produced to allow the impedance line measurement

# PCB marking

The PCB manufacturer must indicate clearly his name. (An abbreviation is admitted)

The PCB manufacturer must indicate the manufacturing data code (e.g. 0641 for week 41 year 2006)

No Rohs marking is required unless specified. Need

# PCB milling diagram

Please insert diagram or refer to PCB milling or drilling drawing (gerber file, ODB++ file …)

# PCB Drill schedule

Please insert table or refer to PCB Drill schedule (gerber file, ODB++ file …)

Refer Gerber file: drill-drawing-layer1-2.pho & drill-drawing-layer1-2.rep